3GPP TR 21.905 V10.3.0 (2011-03)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Vocabulary for 3GPP Specifications

(Release 10)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Keywords

GSM, UMTS, LTE, Vocabulary

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2011, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	5

2	References	5

3	Terms and definitions	6

0-9	6

A	6

B	8

C	9

D	11

E	12

F	13

G	14

H	15

I	15

J	17

K	17

L	17

M	18

N	20

O	21

P	22

Q	24

R	24

S	27

T	30

U	32

V	33

W	34

X	34

Y	34

Z	34

4	Abbreviations	34

0-9	34

A	34

B	36

C	37

D	39

E	40

F	41

G	42

H	42

I	43

J	44

K	45

L	45

M	46

N	47

O	48

P	49

Q	51

R	51

S	53

T	55

U	57

V	58

W	58

X	58

Y	59

Z	59

5	Equations	60

Annex A:	Change history	62

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The purpose of this report is to identify specialist technical terms used within the 3GPP project for the purposes of specifying service requirements. The motivations for this are:

-	To ensure that editors use terminology that is consistent across specifications.

-	To provide a reader with convenient reference for technical terms that are used across multiple documents.

-	To prevent inconsistent use of terminology across documents.

This document is a collection of terms, definitions and abbreviations related to the baseline documents defining 3GPP objectives and systems framework. This document provides a tool for further work on 3GPP technical documentation and facilitates their understanding.

The terms, definitions and abbreviations as given in this document are either imported from existing documentation (ETSI, ITU or elsewhere) or newly created by 3GPP experts whenever the need for precise vocabulary was identified.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	Void

[2]	3GPP TS 25.990: "Technical Specification Group (TSG) RAN; Vocabulary ".

[3]	“The Path towards UMTS - Technologies for the Information Society” – Report #2, UMTS Forum.

[4]	3GPP TS 23.122: "Non-Access-Statum (NAS) functions related to Mobile Station (MS) in idle mode".

[5]	ETSI TR 180 000: "NGN terminology".

[6]	IEC 60050-161: "International Electrotechnical Vocabulary - Chapter 161: Electromagnetic compatibility".

[7]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[8]	3GPP TS 23.101: "General Universal Mobile Telecommunications System (UMTS) architecture".

# 3	Terms and definitions

## 0-9

1.8V technology Smart Card: A Smart Card operating at 1.8V ± 10% and 3V ± 10%.

1.8V technology Terminal: A terminal operating the Smart Card - Terminal interface at 1.8V ± 10% and 3V ± 10%.

3GPP Generic User Profile (GUP): The 3GPP Generic User Profile is the collection of user related data which affects the way in which an individual user experiences services and which may be accessed in a standardised manner.

3GPP system: the telecommunication system standardised by the 3GPP consisting of a core network and a radio access network that may be either GERAN or UTRAN, or both.

3GPP System core network: refers in this specification to an evolved GSM core network infrastructure.

3GPP System coverage: see coverage area.

3GPP System IC Card: An IC card (or 'smartcard') of defined electromechanical specification which contains at least one USIM.

3GPP System mobile termination: part of the 3GPP System Mobile Station which provides functions specific to the management of the radio interface (Um).

3GPP-WLAN Interworking: Used to generically refer to interworking between the 3GPP system and the WLAN family of standards.

3V technology Smart Card: A Smart Card operating at 3V± 10% and 5V ± 10%.

3V technology Terminal: A terminal operating the Smart Card - Terminal interface at 3V-± 10% and 5V ± 10%.

## A

A/Gb mode: mode of operation of the MS when connected to the Core Network via GERAN and the A and/or Gb interfaces.

Acceptable Cell: A cell that the UE may camp on to make emergency calls. It must satisfy certain conditions.

Access conditions: A set of security attributes associated with a file.

Access delay: The value of elapsed time between an access request and a successful access (source: ITU-T X.140).

Access Stratum: functional grouping consisting of the parts in the infrastructure and in the user equipment and the protocols between these parts being specific to the access technique (i.e. the way the specific physical media between the User Equipment and the Infrastructure is used to carry information).

Note:	For full definition, see 23.101 [8], clause 6.2.

Access Stratum SDU (Service Data Unit): Unit of data transferred over the access stratum SAP (Service Access Point) in the Core Network or in the User Equipment.

Access protocol: A defined set of procedures that is adopted at an interface at a specified reference point between a user and a network to enable the user to employ the services and/or facilities of that network (source: ITU-T I.112).

Accounting: The process of apportioning charges between the Home Environment, Serving Network and User.

Accuracy: A performance criterion that describes the degree of correctness with which a function is performed. (The function may or may not be performed with the desired speed.) (source: ITU-T I.350).

Active communication: a UE is in active communication when it has a CS connection established. For PS active communication is defined by the existence of one or more Activated PDP contexts. Either one or both of the mentioned active communications may occur in the UE.

Active Set: Set of radio links simultaneously involved in a specific communication service between an UE and a UTRAN.

Adjacent Channel Leakage power Ratio (ACLR): The ratio of the average power centered on the assigned channel frequency to the average power centered on an adjacent channel frequency. In both cases the average power is measured with a filter that has Root Raised Cosine (RRC) filter response with roll-off = 0.22 and a bandwidth equal to the chip rate.

Air Interface User Rate: The user rate between Mobile Termination and IWF. For T services it is the maximum possible AIUR not including padding. For NT services it is the maximum possible AIUR.

ALCAP: Generic name for the transport signalling protocols used to set-up and tear-down transport bearers.

Allowable PLMN: A PLMN which is not in the list of forbidden PLMN in the UE.

Allowed CSG list: A list stored in the UE containing the CSG identities and associated PLMN identities of the CSGs to which the subscriber belongs.

Ancillary equipment: Equipment (apparatus), used in connection with a receiver, transmitter or transceiver is considered as an ancillary equipment (apparatus) if:

-	the equipment is intended for use in conjunction with a receiver, transmitter or transceiver to provide additional operational and/or control features to the radio equipment, (e.g. to extend control to another position or location); and

-	the equipment cannot be used on a stand alone basis to provide user functions independently of a receiver, transmitter or transceiver; and

-	the receiver, transmitter or transceiver to which it is connected, is capable of providing some intended operation such as transmitting and/or receiving without the ancillary equipment (i.e. it is not a sub unit of the main equipment essential to the main equipment basic functions).

Applet: A small program that is intended not to be run on its own, but rather to be embedded inside another application

Application: an application is a service enabler deployed by service providers, manufacturers or users. Individual applications will often be enablers for a wide range of services. (UMTS Forum report #2) [3]

Applications / Clients: These are services, which are designed using service capability features.

Application Dedicated File (ADF): an application DF is the entry point to an application on the UICC.

Application Interface: Standardised Interface used by application/clients to access service capability features.

Application protocol: The set of procedures required by the application.

ASCI Generic name to identify the services VGCS, VBS and eMLPP.

Authentication: A property by which the correct identity of an entity or party is established with a required assurance. The party being authenticated could be a user, subscriber, home environment or serving network.

Available PLMN: A PLMN where the UE has found a cell that satisfies certain conditions.

Average power: The thermal power as measured through a root raised cosine filter with roll-off = 0.22 and a bandwidth equal to the chip rate of the radio access mode. The period of measurement shall be one power control group (timeslot) unless otherwise stated.

## B

Band category: A group of operating bands for which the same MSR scenarios apply

Base Station: A base station is a network element in radio access network  responsible for radio transmission and  reception in one or more cells to or from the  user equipment. A base station can have an integrated antenna or be connected to an antenna by feeder cables. In UTRAN it terminates the Iub interface towards the RNC. In GERAN it terminates the Abis interface towards the BSC.

Baseline capabilities: Capabilities that are required for a service-less UE to operate within a network. The baseline capabilities for a UE include the capabilities to search for, synchronise with and register (with authentication) to a network. The negotiation of the UE and the network capabilities, as well as the maintenance and termination of the registration are also part of the required baseline capabilities.

Base Station Controller: This equipment in the BSS is in charge of controlling the use and the integrity of the radio resources.

Base station receive period: The time during which the base station is receiving data subframes or UpPTS.

Base Station RF bandwidth: The bandwidth in which a Base Station transmits and receives multiple carriers and/or RATs simultaneously

Base Station RF bandwidth edge: The frequency of one of the edges of the Base Station RF bandwidth

Base Station Subsystem: Either a full network or only the access part of a GERAN offering the allocation, release and management of specific radio resources to establish means of connection between an MS and the GERAN.
A Base Station Subsystem is responsible for the resources and transmission/reception in a set of cells.

Baseline Implementation Capabilities: Set of Implementation capabilities, in each technical domain, required to enable a UE to support the required Baseline capabilities.

Basic OR Basic Optimal Routeing

Basic telecommunication service: This term is used as a common reference to both bearer services and teleservices.

Bearer: A information transmission path of defined capacity, delay and bit error rate, etc.

Bearer capability: A transmission function which the UE requests to the network.

Bearer independent protocol: (UICC) Mechanism by which the ME provides the (U)SIM applications on the UICC with access to the data bearers supported by the ME and the network.

Bearer service: A type of telecommunication service that provides the capability of transmission of signals between access points.

Best effort QoS: The lowest of all QoS traffic classes. If the guaranteed QoS cannot be delivered, the bearer network delivers the QoS which can also be called best effort QoS.

Best effort service: A service model which provides minimal performance guarantees, allowing an unspecified variance in the measured performance criteria.

Billing: A function whereby CDRs generated by the charging function are transformed into bills requiring payment.

Broadcast: A value of the service attribute "communication configuration", which denotes unidirectional distribution to all users (source: ITU-T I.113).

Byte code: (UICC) A hardware machine independent representation of a primitive computer operation that serves as an instruction to a software program called an interpreter or a virtual machine that simulates the hypothetical computer’s central processing unit. code generated by a Java compiler and executed by the Java interpreter.

## C

Cable, Connector, and Combiner Losses (Transmitter) (dB): The combined losses of all transmission system components between the transmitter output and the antenna input (all losses in positive dB values).

Cable, Connector, and Splitter Losses (Receiver) (dB): The combined losses of all transmission system components between the receiving antenna output and the receiver input.

CAC (Connection Admission Control): A set of measures taken by the network to balance between the QoS requirements of new connections request and the current network utilisation without affecting the grade of service of existing/already established connections.

Call: a logical association between several users (this could be connection oriented or connection less).

Carrier: The modulated waveform conveying the E-UTRA, UTRA or GSM/EDGE physical channels

Carrier frequency: center frequency of the cell

Camped on a cell: The UE is in idle mode and has completed the cell selection/reselection process and has chosen a cell. The UE monitors system information and  (in most cases) paging information. Note that the services may be limited, and that the PLMN may not be aware of the existence of the UE within the chosen cell.

Capability Class: A piece of information which indicates general 3GPP System mobile station characteristics (e.g. supported radio interfaces,...) for the interest of the network.

Card session: A link between the card and the external world starting with the ATR and ending with a subsequent reset or a deactivation of the card.

CBS DRX cycle: The time interval between successive readings of  BMC messages.

Cell: Radio network object that can be uniquely identified by a User Equipment from a (cell) identification that is broadcasted over a geographical area from one UTRAN Access Point. A Cell is either FDD or TDD mode.

Cell Radio Network Temporary Identifier (C-RNTI): The  C-RNTI is a UE identifier allocated by a controlling RNC and it is unique within one cell controlled by the allocating CRNC. C-RNTI can be reallocated when a UE accesses a new cell with the cell update procedure.

Cellular Text telephone Modem (CTM):  A modulation and coding method intended for transmission of text in voice channels for the application of real time text conversation.

Channel bandwidth: The RF bandwidth supporting a single RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

Channel edge: The lowest and highest frequency of the carrier, separated by the channel bandwidth.

Chargeable Event: An activity utilising telecommunications network infrastructure and related services for user to user communication (e.g. a single call, a data communication session or a short message), or for user to network communication (e.g. service profile administration), or for inter-network communication (e.g. transferring calls, signalling, or short messages), or for mobility (e.g. roaming or inter-system handover), which the network operator wants to charge for. The cost of a chargeable event may cover the cost of sending, transporting, delivery and storage. The cost of call related signalling may also be included.

Charged Party: A user involved in a chargeable event who has to pay parts or the whole charges of the chargeable event, or a third party paying the charges caused by one or all users involved in the chargeable event, or a network operator.

Charging: A function whereby information related to a chargeable event is formatted and transferred in order to make it possible to determine usage for which the charged party may be billed.

Charging Data Record (CDR): A formatted collection of information about a chargeable event (e.g. time of call set-up, duration of the call, amount of data transferred, etc) for use in billing and accounting. For each party to be charged for parts of or all charges of a chargeable event a separate CDR shall be generated, i.e more than one CDR may be generated for a single chargeable event, e.g. because of its long duration, or because more than one charged party is to be charged.

Cipher key: A code used in conjunction with a security algorithm to encode and decode user and/or signalling data.

Closed group: A group with a pre-defined set of members. Only defined members may participate in a closed group.

Closed Subscriber Group (CSG): A Closed Subscriber Group identifies subscribers of an operator who are permitted to access one or more cells of the PLMN but which have restricted access (CSG cells).

Coded Composite Transport Channel: A data stream resulting from encoding and multiplexing of one or several transport channels.

Common Channel: A Channel not dedicated to a specific UE.

Confidentiality: The avoidance of disclosure of information without the permission of its owner.

Connected Mode: Connected mode is the state of User Equipment switched on and an RRC connection established.

Connection: A communication channel between two or more end-points (e.g. terminal, server etc.).

Connection mode: The type of association between two points as required by the bearer service for the transfer of information. A bearer service is either connection-oriented or connectionless. In a connection oriented mode, a logical association called connection needs to be established between the source and the destination entities before information can be exchanged between them. Connection oriented bearer services lifetime is the period of time between the establishment and the release of the connection. In a connectionless mode, no connection is established beforehand between the source and the destination entities; the source and destination network addresses need to be specified in each message. Transferred information cannot be guaranteed of ordered delivery. Connectionless bearer services lifetime is reduced to the transport of one message.

Connectionless (for a bearer service): In a connectionless bearer, no connection is established beforehand between the source and the destination entities ; the source and destination network addresses need to be specified in each message. Transferred information cannot be guaranteed of ordered delivery. Connectionless bearer services lifetime is reduced to the transport of one message.

Connectionless service: A service which allows the transfer of information among service users without the need for end-to-end call establishment procedures (source: ITU-T I.113).

Continuous phenomena (continuous disturbance): Electromagnetic disturbance, the effects of which on a particular device or equipment cannot be resolved into a succession of distinct effects (IEC 60050-161 [6]).

Control channel: A logical channel that carries system control information.

Controlling RNC: A role an RNC can take with respect to a specific set of UTRAN access points. There is only one Controlling RNC for any UTRAN access point. The Controlling RNC has the overall control of the logical resources of its UTRAN access point's.

Conversational service: An interactive service which provides for bi-directional communication by means of real-time (no store-and-forward) end-to-end information transfer from user to user (source: ITU-T I.113).

Core network: An architectural term relating to the part of 3GPP System which is independent of the connection technology of the terminal (eg radio, wired).

Core Network Operator: Operator that offers core network services.

Corporate code: Code which when combined with the network and SP codes refers to a unique Corporate.  The code is provided in the GID2 file on the (U)SIM (see Annex A.1.) and is correspondingly stored on the ME.

Corporate code group combination of the Corporate code and the associated SP and network codes.

Corporate personalisation: Allows a corporate customer to personalise MEs that he provides for his employees or customers use so that they can only be used with the company's own (U)SIMs.

Coverage area (of a mobile cellular system): An area where mobile cellular services are provided by that mobile cellular system to the level required of that system.

Coverage area: Area over which a 3GPP System service is provided with the service probability above a certain threshold.

CSG cell: A cell, part of the PLMN, broadcasting a specific CSG Identity. A CSG cell is accessible by the members of the closed subscribers group for that CSG Identity. All the CSG cells sharing the same identity are identifiable as a single group.

CSG Identity (CSGID): An identity broadcast by a CSG cell or cells and used by the UE to facilitate access for authorised members of the associated Closed Subscriber Group.

CSG Indicator: An indication transmitted on the broadcast channel of the CSG cell that allows the UE to identify such as CSG cell.

CSG manager: A CSG manager can, under the operator's supervision, add, remove and view the list of CSG members.

Current directory: The latest MF or DF selected on the UICC.

Current EF: The latest EF selected.

Current serving cell: This is the cell on which the MS is camped.

## D

Data field: Obsolete term for Elementary File.

Data Object: Information coded as TLV objects, i.e. consisting of a Tag, a Length and a Value part.

Dedicated Channel:  A channel dedicated to a specific UE.

De-personalisation: Is the process of deactivating the personalisation so that the ME ceases to carry out the verification checks.

Dedicated File (DF): A file containing access conditions and, optionally, Elementary Files (EFs) or other Dedicated Files (DFs).

Delivered QoS: Actual QoS parameter values with which the content was delivered over the lifetime of a QoS session.

Demand service: A type of telecommunication service in which the communication path is established almost immediately, in response to a user request effected by means of user-network signalling (source: ITU-T I.112).

Dependability: A performance criterion that describes the degree of certainty (or surety) with which a function is performed regardless of speed or accuracy, but within a given observational interval (source: ITU-T I.350).

Destination user: Entity to which calls to the General Packet Radio Service (GPRS) are directed.

Directory: General term for the MF or a DF on the UICC.

Directory Number: A string consisting of one or more of the characters from the set {0, 1, 2, 3, 4, 5, 6, 7, 8, 9, *, #, a, b, c} associated with a nature of address indicator and number plan indicator.  When using the public MMI for the control of supplementary services however, * and # cannot be part of any SC or SI field.

NOTE 1:	No such restriction on the SC and SI fields exists when using other (e.g. menu-driven) MMI for the control of supplementary services.

NOTE 2:	When using the public MMI, certain limitations on the use of one and two digit directory numbers may apply.  The use of other MMI can remove these restrictions.

NOTE 3:	This definition is not intended to require the support of all these characters in the MMI itself.

Distribution service: Service characterised by the unidirectional flow of information from a given point in the network to other (multiple) locations (source: ITU-T I.113).

DL RS power: The resource element power of Downlink Reference Symbol.

Domain: The highest-level group of physical entities. Reference points are defined between domains.

Domain Specific Access Control: Access control functionality for access barring in either domain (i.e. CS domain or PS domain).

Donor coupling loss: the coupling loss between the repeater and the donor base station.

Donor network: The subscription network from which a number is ported in the porting process. This may or may not be the number range owner network.

Downlink: Unidirectional radio link for the transmission of signals from a UTRAN access point to a UE. Also in general the direction from Network to UE.

Downlink operating band: The part of the operating band designated for downlink.

Downlink Pilot Timeslot: Downlink part of the special subframe (for TDD operation)

Drift RNS: The role an RNS can take with respect to a specific connection between a UE and UTRAN. An RNS that supports the Serving RNS with radio resources when the connection between the UTRAN and the User Equipment need to use cell(s) controlled by this RNS is referred to as Drift RNS.

## E

Element Manager: Provides a package of end-user functions for management of a set of closely related types of network elements. These functions can be divided into two main categories.

Element Management Functions: Set of functions for management of network elements on an individual basis. These are basically the same functions as supported by the corresponding local terminals.

Elementary File (EF): A file containing access conditions and data and no other files on the UICC.

Elementary procedure (EP): The RANAP, RNSAP, NBAP, S1AP, X2AP, PCAP, HNBAP, LPPa, RNA, RUA, RETAP and TMAAP protocols consist of elementary procedures (EPs).

An EP consists of an initiating message and possibly a response message.

Three kinds of EP are used:

-	Class 1: Elementary Procedures with response (success or failure).

-	Class 2: Elementary Procedures without response.

-	Class 3: Elementary Procedures with possibility of multiple responses (RANAP only).

For Class 1 EPs, the types of responses can be as follows:

Successful

-	A signalling message explicitly indicates that the elementary procedure has been successfully completed with the receipt of the response.

Unsuccessful

-	A signalling message explicitly indicates that the EP failed.

Class 2 EPs are considered always successful.

Class 3 EPs have one or several response messages reporting both successful and unsuccessful outcome of the requests, and temporary status information about the requests. This type of EP only terminates through response(s) or EP timer expiry.

End-User: An End-User is an entity (typically a user), associated with one or multiple subscriptions through identities  (e.g. IMSIs, MSISDNs, IMPIs, IMPUs and application-specific identities). In the 3GPP system an End-User is characterised by an End-User Identity.

End-User Identity (EUI): An End-User Identity is an identity that uniquely characterises an End-User in the 3GPP system. An End-User Identity is mainly intended for administration purposes of the operator.

Enterprise Systems: Information Systems that are used in the telecommunication organisation but are not directly or essentially related to the telecommunications aspects (Call Centre's, Fraud Detection and Prevention Systems, Invoicing etc).

Equivalent HPLMN / Equivalent Home PLMN (EHPLMN): Any of the PLMN entries contained in the Equivalent HPLMN list.

Equivalent HPLMN list: To allow provision for multiple HPLMN codes, PLMN codes that are present within this list shall replace the HPLMN code derived from the IMSI for PLMN selection purposes. This list is stored on the USIM and is known as the EHPLMN list. The EHPLMN list may also contain the HPLMN code derived from the IMSI. If the HPLMN code derived from the IMSI is not present in the EHPLMN list then it shall be treated as a Visited PLMN for PLMN selection purposes.

E-UTRAN Radio Access Bearer (E-RAB): An E-RAB uniquely identifies the concatenation of an S1 Bearer and the corresponding Data Radio Bearer. When an E-RAB exists, there is a one-to-one mapping between this E-RAB and an EPS bearer of the Non Access Stratum as defined in [7].

Essential UE Requirement (Conditional): Requirement which has to be implemented under certain Service conditions.  e.g. AMR codec in UE which supports speech service

Essential UE Requirement (Unconditional): Requirement which has to be implemented in any 3G UE in order to exist in and communicate with 3G network (e.g. Chiprate of 3.84Mcps).

Evolved Packet Core: Is a framework for an evolution or migration of the 3GPP system to a higher-data-rate, lower-latency, packet-optimized system that supports, multiple RATs.

Evolved Packet System: Is an evolution of the 3G UMTS characterized by higher-data-rate, lower-latency, packet-optimized system that supports multiple RATs.  The Evolved Packet System comprises the Evolved Packet Core together with the evolved radio access network (E-UTRA and E-UTRAN).

Evolved UTRA: Evolved UTRA is an evolution of the 3G UMTS radio-access technology towards a high-data-rate, low-latency and packet-optimized radio-access technology.

Evolved UTRAN: Evolved UTRAN is an evolution of the 3G UMTS radio-access network towards a high-data-rate, low-latency and packet-optimized radio-access network.

Explicit Diversity Gain (dB): The effective gain achieved using diversity techniques.

Extra SDU delivery probability: The ratio of total (unrequested) extra service data units (SDUs) to total service data units received by a destination user in a specified sample (source: ITU-T X.140).

NOTE:	the term "user information unit" has been replaced by the term "service data unit".

## F

File: A named and hierarchically-classified data set on the UICC.

File identifier (FID): The 2-byte name of a file or a directory on the UICC.

Fixed Network User Rate: The user rate between IWF and the fixed network.

FC (Flow Control): A set of mechanisms used to prevent the network from becoming overloaded by regulating the input rate transmissions.

Flexible Layer One (FLO): GERAN feature that allows the channel coding of the layer one to be configured at call setup.

Fixed Mobile Convergence (FMC): In a given network configuration, the capabilities that provide service and application to the end-user irrespective of the fixed or mobile access technologies and independent of user's location. In the NGN environment, it means to provide NGN services to end-users regardless of the access technology.

Framework: A framework defines a set of Application Programming Interface (API) classes for developing applications and for providing system services to those applications.

Frequency layer: set of cells with the same carrier frequency.

Functional group: A set of functions that may be performed by a single equipment (source: ITU-T I.112).

## G

Geographical routing: The conversion of the PDU’s geographical area definition, which specifies the area in which the PDU will be broadcast, into an equivalent radio coverage map.

GERAN Radio Network Temporary Identifier (G-RNTI): G-RNTI is an MS identifier which is allocated by the Serving BSC and is unique within this SBSC. It is allocated for all MSs having an RRC connection.  The G-RNTI is always reallocated when the Serving BSC for the RRC connection is changed and deallocated when the RRC connection is released. The G-RNTI is also used at RLC/MAC during contention resolution.

GPRS MS: An MS capable of GPRS services is a GPRS MS.

Group: A set of members allowed to participate in the group call service. The group is defined by a set of rules that identifies a collection of members implicitly or explicitly. These rules may associate members for the purpose of participating in a group call, or may associate members who do not participate in data transfer but do participate in management, security, control, or accounting for the group.

Group call: The relationship that exists between the members of a group for the purpose of transferring data. More than one group call may exist in a group. A group call establishes an active group.

Group call initiator: A member (or third party) authorised to initiate a group call. More than one member may initiate group calls.

Group call participant: A member of a group participating in a particular group call at a given time.

Group call server: A logical entity that provides the group call service to the members.

Group call service: A PTM service in which a relationship exists between participants of the group, and in which a single data unit transmitted by a source participant is received by multiple destination participants; it is a one-in, many-out service.

Group controller: The member (or third party) responsible for the group creation and membership control.

GSM/EDGE Radio Access Network: GERAN is a conceptual term identifying that part of the network which consists of BSCs and BTSs between A/Gb or Iu and Um interfaces.

GSM BSS: refers in this specification to the GSM/GPRS access network.

GSM core network: refers in this specification to the GSM NSS and GPRS backbone infrastructure.

GSM coverage: an area where mobile cellular services are provided in accordance with GSM standards

GSM session: That part of the card session dedicated to the GSM operation.

Guaranteed service: A service model which provides highly reliable performance, with little or no variance in the measured performance criteria.

## H

Handoff Gain/Loss (dB): This is the gain/loss factor (+ or -) brought by handoff to maintain specified reliability at the cell boundary.

Handover: The transfer of a user’s connection from one radio channel to another (can be the same or different cell).

Handover: The process in which the radio access network changes the radio transmitters or radio access mode or radio system used to provide the bearer services, while maintaining a defined bearer service QoS.

Hard Handover: Hard handover is a category of handover procedures where all the old radio links in the UE  are abandoned before the new radio links are established.

Heterogeneous Network:  a 3GPP access network consisting of multiple cells with different characteristics (e.g., for the case of E-UTRA: a variety of e-NodeBs, Home e-NodeBs, e-UTRA Relays).

HE-VASP: Home Environment Value Added Service Provider. This is a VASP that has an agreement with the Home Environment to provide services. The Home Environment provides services to the user in a managed way, possibly by collaborating with HE-VASPs, but this is transparent to the user. The same service could be provided by more than one HE-VASP and each HE-VASP can provide more than one service.

Home Environment: responsible for overall provision and control of the Personal Service Environment of its subscribers.

HNB Name: The HNB Name is a broadcast string in free text format that provides a human readable name for the Home NodeB/eNodeB.

Home PLMN: This is a PLMN where the MCC and MNC of the PLMN identity match the MCC and MNC of the IMSI. Matching criteria are defined in TS 23.122.

Hybrid cell: A cell broadcasting a CSG indicator set to false and a specific CSG identity. This cell is accessible as a CSG cell by UEs which are members of the CSG and as a normal cell by all other UEs.

## I

IC Card: A card holding an Integrated Circuit containing subscriber, end user, authentication and/or application data for one or more applications.

IC card SIM: Obsolete term for ID-1 SIM.

ICS proforma: A document, in the form of a questionnaire, which when completed for an implementation or system becomes an ICS.

ID-000 SIM: A UICC having the form on an ID-000 card (see ISO 7816-1 [24]) that contains a SIM application.

ID-1 SIM: A UICC having the format of an ID-1 card (see ISO 7816-1 [24]) that contains a SIM.

Idle mode: The state of UE switched on but which does not have any established RRC connection.

IP Flow Mobility: distribution of IP flows on a UE between 3GPP and WLAN available accesses based on the different characteristics of the IP flows, the operator policies and the capabilities of the available accesses.

Implementation capability: A capability that relates to a particular technical domain. Examples: a spreading factor of 128 (in the domain of the physical layer); the A5 algorithm; a 64 bit key length  (in the domain of security); a power output of 21 dBm (in the domain of transmitter performance); support of AMR Codec (in the domain of the Codec); support of CHV1 (in the domain of the USIM).

Implementation Conformance Statement (ICS): A statement made by the supplier of an implementation or system claimed to conform to a given specification, stating which capabilities have been implemented. The ICS can take several forms: protocol ICS, profile ICS, profile specific ICS, information object ICS, etc.

Information Data Rate: Rate of the user information, which must be transmitted over the Air Interface. For example, output rate of the voice codec.

Initial paging information: This information indicates if the UE needs to continue to read more paging information and eventually receive a page message.

Initial paging occasion: The paging occasion the UE uses as starting point for its paging DRX cycle.

Integrity: (in the context of security) The avoidance of unauthorised modification of information.

Inter-cell handover: A handover between different cells. An inter-cell handover requires network connections to be altered.

Inter PLMN handover: Handover between different PLMNs, ie having different MCC-MNC.

Inter system handover: Handover between networks using different radiosystems , e.g. UMTS – GSM.

Interactive service: A service which provides the means for bi-directional exchange of information between users. Interactive services are divided into three classes of services: conversational services, messaging services and retrieval services (source: ITU-T I.113).

Interface: The common boundary between two associated systems (source: ITU-T I.112).

International Mobile Station Equipment Identity (IMEI): An "International Mobile Station Equipment Identity" is a unique number which shall be allocated to each individual mobile station equipment  in the PLMN and shall be unconditionally implemented by the MS manufacturer.

International mobile user number (IMUN): The International Mobile User Number is a diallable number allocated to a 3GPP System user.

Interference Signal Code Power (ISCP): Given only interference power is received, the average power of the received signal after despreading and combining.

Interpreter:  A software program that simulates a hypothetical computer by performing the operations defined by the instructions of this computer.(see also 'byte code' and 'virtual machine').

Interworking WLAN (I-WLAN): A WLAN that interworks with a 3GPP system.

Intra-cell handover: A handover within one sector or between different sectors of the same cell. An intra-cell handover does not require network connections to be altered.

Intra PLMN handover: Handover within the same network, ie having the same MCC-MNC regardless of radio access system.

Note:	this includes the case of UMTS <>GSM handover where MCC-MNC are the same in both cases.

IP-Connectivity Access Network (IP-CAN): The collection of network entities and interfaces that provides the underlying IP transport connectivity between the UE and the IMS entities. An example of an "IP-Connectivity Access Network" is GPRS.

IP-Connectivity Access Network bearer (IP-CAN bearer): The data communications bearer provided by the IP-Connectivity Access Network. When using GPRS, the IP-Connectivity Access Network bearers are provided by PDP Contexts.

IRP Information Model: An IRP Information Model consists of an IRP Information Service and a Network Resource Model (see below for definitions of IRP Information Service and Network Resource Model).

IRP Information Service: An IRP Information Service describes the information flow and support objects for a certain functional area, e.g. the alarm information service in the fault management area. As an example of support objects, for the Alarm IRP there is the alarm record and alarm list.

IRP Solution Set: An IRP Solution Set is a mapping of the IRP Information Service to one of several technologies (CORBA/IDL, SNMP/SMI, CMIP/GDMO, etc.). An IRP Information Service can be mapped to several different IRP Solution Sets. Different technology selections may be done for different IRPs.

Inter System Change: a change of radio access between different radio access technologies such as GSM and UMTS.

IMS Credentials (IMC): A set of IMS security data and functions for IMS access by a terminal that does not support any 3GPP access technology.. The IMC is not including an ISIM or a USIM. The IMC is not used if ISIM or USIM is present.

IMS Multimedia Telephony: A service that allows multimedia conversational communications between two or more users. It provides real time bidirectional conversational transfer of media, e.g. speech, video, text or other types of data. The IMS multimedia telephony service includes Supplementary Services and takes account of regulatory requirements.

IMS SIM (ISIM): An application residing on the UICC that provides access to IP Multimedia Services.

Iu: Interconnection point between an RNC or a BSC and a 3G Core Network. It is also considered as a reference point.

Iu-flex:  Routing functionality for intra domain connection of RAN nodes to multiple CN nodes.

Iu mode: mode of operation of the MS when connected to the Core Network via GERAN or UTRAN and the Iu interface.

Iub: Interface between an RNC and a Node B.

Iur: A logical interface between two RNC. Whilst logically representing a point to point link between RNC, the physical realisation may not be a point to point link.

## J

<void>

## K

Key pair: Key pairs are matching private and public keys. If a block of data is encrypted using the private key, the public key from the pair can be used to decrypt it. The private key is never divulged to any other party, but the public key is available, e.g. in a certificate.

## L

Local Service: Services, which are provided by current roamed to network that are not HE services. The same service can be provided by a network as a local service to inbound roamers and as a HE service to the subscribers of this network.

Local IP Access (LIPA): Allows an IP-capable UE connected via a H(e)NB direct access to other IP-capable devices in the local residential/corporate IP network.

Localised Service Area (LSA): A LSA is an operator-defined group of cells, for which specific access conditions apply. This may correspond to an area in which the Core Network offers specific services. A LSA may be defined within a PLMN or globally. Therefore, a LSA may offer a non-contiguous radio coverage.

Location Registration (LR): The UE registers its presence in a registration area, for instance regularly or when entering a new registration area.

Logical Channel: A logical channel is an information stream dedicated to the transfer of a specific type of information over the radio interface. Logical Channels are provided on top of the MAC layer.

Logical Channel (UICC): A command/response communication context multiplexed on the physical channel between the ME and the UICC.

Logical Model: A Logical Model defines an abstract view of a network or network element by means of information objects representing network element, aggregations of network elements, the topological relationship between the elements, endpoints of connections (termination points), and transport entities (such as connections) that transport information between two or more termination points.
The information objects defined in the Logical Model are used, among others, by connection management functions. In this way a physical implementation independent management is achieved.

Logical O&M: Logical O&M is the signalling associated with the control of logical resources (channels, cells,) owned by the RNC but physically implemented in the Node B. The RNC controls these logical resources. A number of O&M procedures physically implemented in Node B impact on the logical resources and therefore require an information exchange between RNC and Node B. All messages needed to support this information exchange are classified as Logical O&M forming an integral part of NBAP.

Lower RF bandwidth edge: The frequency of the lower edge of the Base Station RF bandwidth, used as a frequency reference point for transmitter and receiver requirements

LSA exclusive access cell: A UE may only camp on this cell if the cell belongs to the LSAs to which the user has subscribed. Nevertheless, if no other cells are available, the UE of non-LSA users may originate emergency calls from this cell.

LSA only access: When LSA only access applies to the user, the UE can only access cells that belong to the LSAs to which the user has subscribed. Outside the coverage area of the subscribed LSAs, the UE may camp on other cells and limited services apply.

LSA preferential access cell: A LSA preferential access cell is a cell which is part of the LSA. UEs of users that have subscribed to a LSA of a LSA-preferential-access cell have higher priority to resources than non-LSA users in the same cell.

## M

Macro cells: "Macro cells" are outdoor cells with a large cell radius.

Macro diversity handover: "Macro diversity" is a operation state in which a User Equipment simultaneously has radio links with two or more UTRAN access points for the sole aim of improving quality of the radio connection or providing seamless.

Management Infrastructure: The collection of systems (computers and telecommunications) a 3GPP System Organisation has in order to manage a 3GPP System.

Mandatory UE Requirement: Regulatory requirement which is applicable to 3G UEs.   It is determined by each country/region and beyond the scope of 3GPP specification (e.g. spurious emission in UK).

Master File (MF): The root directory of the file system hierarchy on the UICC.

Maximum Base Station RF bandwidth: The maximum RF bandwidth supported by a BS within an operating band.

Maximum output Power: For UE, this  is a measure of the maximum power supported by the UE (i.e. the actual power as would be measured assuming no measurement error) (TS 25.101). For FDD BS, the mean power level per carrier of the base station measured at the antenna connector in a specified reference condition (TS 25.104). For TDD BS this refers to the measure of power when averaged over the transmit timeslot at the maximum power setting (TS 25.105).  For LTE: the mean power level per carrier of the base station measured at the antenna connector in a specified reference condition.

Maximum possible AIUR: The highest possible AIUR that the multiple TCH/F can provide, e.g. 2 TCH/F using TCH/F9.6 provides a maximum possible AIUR of 19,2 kbit/s.

Maximum throughput: maximum achievable throughput for a reference measurement channel.

Maximum total output power: sum of the power of all carriers available at the antenna connector for a specified reference condition.

Maximum Transmitter Power Per Traffic Channel (dBm): The maximum power at the transmitter output for a single traffic channel.

MBMS-service-associated signalling: When M2AP messages associated to one MBMS service uses the MBMS-service-associated logical M2-connection for association of the message to the respective MBMS service in eNB and EPC.

Mean bit rate: A measure of throughput. The average (mean) bit rate available to the user for the given period of time (source: ITU-T I.210).

Mean power: When applied to E-UTRA transmission this is the power measured in the operating system bandwidth of the carrier. The period of measurement shall be at least one subframe (1ms) unless otherwise stated.

Mean transit delay: The average transit delay experienced by a (typically) large sample of PDUs within the same service category.

Measurement bandwidth: The bandwidth in which an emission level is specified.

Medium Access Control: A sub-layer of radio interface layer 2 providing unacknowledged data transfer service on logical channels and access to transport channels.

Messaging service: An interactive service which offers user-to-user communication between individual users via storage units with store-and-forward, mailbox and/or message handling, (e.g., information editing, processing and conversion) functions (source: ITU-T I.113).

MExE Classmark: A MExE classmark identifies a category of MExE UE supporting MExE functionality with a minimum level of processing, memory, display, and interactive capabilities. Several MExE classmarks may be defined to differentiate between the functionalities offered by different MExE UEs. A MExE application or applet defined as being of a specific MExE Classmark indicates that it is supportable by a MExE UE of that Classmark.

MExE executable: An executable is an applet, application, or executable content, which conforms to the MExE specification and may execute on the ME.

MExE server: A node supporting MExE services in the MExE service environment.

MExE service: a service enhanced (or made possible) by MExE technology.

MExE service environment: Depending on the configuration of the PLMN, the operator may be able to offer support to MExE services in various ways. Examples of possible sources are from traditional GSM nodes, IN nodes, operator-specific nodes, operator franchised nodes and services provider nodes, together with access to nodes external (i.e. vendor-specific) to the PLMN depending on the nature of the MExE service. These nodes are considered to constitute the MExE service environment. The MExE service environment shall support direct MExE UE to MExE UE interaction of MExE services.

MExE service provider: an organisation which delivers MExE services to the subscriber. This is normally the PLMN operator, but could be an organisation with MExE responsibility (which may have been delegated by the PLMN operator).

MExE SIM: A (U)SIM application that is capable of storing a security certificate that is accessible using standard mechanisms.

MExE subscriber: The owner of a subscription who has entered into an agreement with a MExE service provider for MExE services.

Micro cells: "Micro cells" are small  cells.

Minimum transmit power: The minimum controlled output power of the TDD BS is when the power control setting is set to a minimum value. Thei si when the power control indicates a miminum transmit output power is required (TS 25.105).

Mobile Equipment (ME): The Mobile Equipment is functionally divided into several entities, i.e.one or more Mobile Terminations (MT) and one or more Terminal Equipments (TE).

Mobile evaluated handover: Mobile evaluated handover (MEHO) is a type of handover triggered by an evaluation made in the mobile.  The mobile evaluates the necessity of handover based on the measured radio environment and based on criteria defined by the network.  When the evaluation meets the hand-off criteria the necessary information is sent from the mobile to the network.  The network then decides on the necessity of the handover based on the reported evaluation result and other conditions, e.g. uplink radio environment and/or availability of network resources, the network may then execute the handover.

Mobile Station (MS): A Mobile Station (MS) corresponds to a User Equipment (UE). See 3GPP TS 24.002.

Mobile number portability: The ability for a mobile subscriber to change subscription network within the same country whilst retaining their original MSISDN(s).

Mobile Termination (MT): The Mobile Termination is the component of the Mobile Equipment (ME) which supports functions specific to management of the PLMN access interface (3GPP or non-3GPP). The MT is realized as a single functional entity..

Mobility: The ability for the user to communicate whilst moving independent of location.

Mobility Management: A relation between the mobile station and the UTRAN that is used to set-up, maintain and release the various physical channels.

MSR Base station: Base Station characterized by the ability of its receiver and transmitter to process two or more carriers in common active RF components simultaneously in a declared RF bandwidth, where at least one carrier is of a different RAT than the other carrier(s).

Multi-carrier transmission configuration: A set of one or more contiguous carriers that a BS is able to transmit simultaneously according to the manufacturer’s specification.

Multi mode terminal: UE that can obtain service from at least one UTRA radio access mode, and one or more different systems such as GSM bands or possibly other radio systems such IMT-2000 family members.

Multicast service: A unidirectional PTM service in which a message is transmitted from a single source entity to all subscribers currently located within a geographical area. The message contains a group identifier indicating whether the message is of interest to all subscribers or to only the subset of subscribers belonging to a specific multicast group.

Multipoint: A value of the service attribute "communication configuration", which denotes that the communication involves more than two network terminations (source: ITU-T I.113).

Multimedia service: Services that handle several types of media such as audio and video in a synchronised way from the user's point of view. A multimedia service may involve multiple parties, multiple connections, and the addition or deletion of resources and users within a single communication session.

## N

Name: A name is an alpha numeric label used for identification of end users and may be portable.

Negotiated QoS: In response to a QoS request, the network shall negotiate each QoS attribute to a level that is in accordance with the available network resources. After QoS negotiation, the bearer network shall always attempt to provide adequate resources to support all of the negotiated QoS profiles.

Network code: MCC and MNC.

Network code group: Same as network code.

Network connection: An association established by a network layer between two users for the transfer of data, which provides explicit identification of a set of network data transmissions and agreement concerning the services to be provided by the set (source: ITU-T X.213 / ISO-IEC 8348).

Network Element: A discrete telecommunications entity which can be managed over a specific interface e.g. the RNC.

Network Manager: Provides a package of end-user functions with the responsibility for the management of a network, mainly as supported by the EM(s) but it may also involve direct access to the network elements. All communication with the network is based on open and well standardized interfaces supporting management of multi-vendor and multi-technology network elements.

Network operator: See PLMN operator.

Network personalisation: Allows the network operator to personalise a ME so that it can only be used with that particular network operator's (U)SIMs.

Network Resource Model: A protocol independent model describing managed objects representing network resources, e.g. an RNC or NodeB.

Network service data unit (NSDU): A unit of data passed between the user and the GPRS network across a Network Service Access Point (NSAP).

Network subset code: digits 6 and 7 of the IMSI.

Network subset code group: Combination of a network subset code and the associated network code.

Network subset personalisation: A refinement of network personalisation, which allows network operators to limit the usage of a ME to a subset of (U)SIMs

Network termination: A functional group on the network side of a user-network interface (source: ITU-T I.112).

Node B: A logical node responsible for radio transmission / reception in one or more cells to/from the User Equipment. Terminates the Iub interface towards the RNC.

Nomadic Operating Mode: Mode of operation where the terminal is transportable but being operated while stationary and may in addition require user co-operation (e.g. close to open spaces, antenna setup...).

Nominal Maximum Output Power: This is the nominal power defined by the UE power class.

Non-Access Stratum: Protocols between UE and the core network that are not  terminated in the UTRAN.

Normal GSM operation: Relating to general, CHV related, GSM security related and subscription related procedures.

Normal mode of operation: The mode of operation into which the ME would have gone if it had no personalisation checks to process.

NTDD: Narrow TDD – the 1.28 Mcps chip rate UTRA-TDD option

Number: A string of decimal digits that uniquely indicates the public network termination point. The number contains the information necessary to route the call to this termination point.

A number can be in a format determined nationally or in an international format. The international format is known as the International Public Telecommunication Number which includes the country code and subsequent digits, but not the international prefix.

Number portability:  A capability that allows a user to retain the same public telecommunication number when changing from one service provider to another. Additional regulatory constraints may apply in different regions.

Number range owner network: The network to which the number range containing the ported number has been allocated.

## O

Occupied bandwidth: The width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage β/2 of the total mean power of a given emission.

Off-Line charging: A charging process where charging information does not affect, in real time, the service rendered.

On-Line Charging: A charging process where charging information can affect, in real time, the service rendered and therefore directly interacts with the session/service control.

One Stop Billing: One bill for all charges incurred using the 3GPP System.

Open group: A group that does not have a pre-defined set of members. Any user may participate in an open group.

Open Service Access: Concept for introducing a vendor independent means for introduction of new services.

Operating band: A frequency range in which E-UTRA operates (paired or unpaired), that is defined with a specific set of technical requirements. Operations System: This abbreviation indicates a generic management system, independent of its location level within the management hierarchy.

Optional UE Requirement: Any other requirements than mandatory UE requirement, essential UE requirement (conditional), essential UE requirement (unconditional). It is totally up to individual manufacturer to decide whether it should be implemented or not (e.g. Network initiated MM connection establishment).

Originating network: The network where the calling party is located.

Orthogonal Channel Noise Simulator a mechanism used to simulate the users or control signals on the other orthogonal channels of a downlink

OSA Interface: Standardised Interface used by application/clients to access service capability features.

Output power (Pout): The mean power of one carrier of the UE, delivered to a load with resistance equal to the nominal load impedance of the transmitter.

## P

Packet: An information unit identified by a label at layer 3 of the OSI reference model (source: ITU-T I.113). A network protocol data unit (NPDU).

Packet data protocol (PDP): Any protocol which transmits data as discrete units known as packets, e.g., IP, or X.25.

Packet transfer mode: Also known as packet mode. A transfer mode in which the transmission and switching functions are achieved by packet oriented techniques, so as to dynamically share network transmission and switching resources between a multiplicity of connections (source: ITU-T I.113).

Padding: One or more bits appended to a message in order to cause the message to contain the required number of bits or bytes.

Paging: The act of seeking a User Equipment.

Paging DRX cycle: The individual time interval between monitoring Paging Occasion for a specific UE

Paging Block Periodicity (PBP): The period of the occurrence of Paging Blocks. (For FDD, PBP = 1).

Paging Message Receiving Occasion: The frame where the UE receives actual paging message.

Paging occasion: The frame where the UE monitors in FDD or the paging block, which consists of several frames, for TDD. For Paging Blocks, the value of Paging Occasion is equal to the first frame of the Paging Block.

Pass band: The frequency range that the repeater operates in with operational configuration. This frequency range can correspond to one or several consecutive nominal channels. If they are not consecutive each subset of channels shall be considered as an individual pass band.

Peak bit rate: A measure of throughput. The maximum bit rate offered to the user for a given time period (to be defined) for the transfer of a bursty signal (source: ITU-T I.210). (The maximum user information transfer rate achievable by a user for a single service data unit transfer.)

Performance: The ability to track service and resource usage levels and to provide feedback on the responsiveness and reliability of the network.

Personal Service Environment: contains personalised information defining how subscribed services are provided and presented towards the user. Each subscriber of the Home Environment has her own Personal Service Environment. The Personal Service Environment is defined in terms of one or more User Profiles.

Personalisation: The process of storing information in the ME and activating the procedures which verify this information against the corresponding information stored in applications on the (U)SIM whenever the ME is powered up or when a  UICC containing network access applications (SIM, USIM, etc.) is inserted, in order to limit the applications with which the ME will operate.

Personalisation entity: Network, network subset, SP, Corporate or (U)SIM to which the ME is personalised

Phonebook: A dataset of personal or entity attributes. The simplest form is a set of name-subscriber phone number pairs as supported by GSM (U)SIMs.

Physical channel data stream: In the uplink, a data stream that is transmitted on one physical channel. In the downlink, a data stream that is transmitted on one physical channel in each cell of the active set.

Physical Channel: In FDD mode, a physical channel is defined by code, frequency and, in the uplink, relative phase (I/Q). In TDD mode, a physical channel is defined by code, frequency, and time-slot.

Pico cells: "Pico cells" are cells, mainly indoor cells, with a radius typically less than 50 metres.

PICH Monitoring Occasion: The time instance where the UE monitors PICH within Paging Occasion.

Pilot Identity: A service specific public address used for initial contact, associated with a group of publicly addressable identities (e.g. E.164 numbers or SIP URI).

PLMN Area: The PLMN area is the geographical area in which a PLMN provides communication services according to the specifications to mobile users. In the PLMN area, the mobile user can set up calls to a user of a terminating network. The terminating network may be a fixed network, the same PLMN, another PLMN or other types of PLMN. Terminating network users can also set up calls to the PLMN. The PLMN area is allocated to a PLMN. It is determined by the service and network provider in accordance with any provisions laid down under national law. In general the PLMN area is restricted to one country. It can also be determined differently, depending on the different telecommunication services, or type of MS. If there are several  PLMNs in one country, their PLMN areas may overlap. In border areas, the PLMN areas of different countries may overlap. Administrations will have to take precautions to ensure that cross border coverage is minimised in adjacent countries unless otherwise agreed.

PLMN Operator: Public Land Mobile Network operator. The entity which offers telecommunications services over an air interface..

Plug-in SIM: A physical form factor of SIM (see ID-000 SIM).

point-to-multipoint service: A service type in which data is sent to ”all service subscribers or a pre-defined subset of all subscribers” within an area defined by the Service Requester.

Point-to-point: A value of the service attribute "communication configuration", which denotes that the communication involves only two network terminations.

Point-to-point service: A service type in which data is sent from a single network termination to another network termination.

Port: A particular interface, of the specified equipment (apparatus), with the electromagnetic environment. For example, any connection point on an equipment intended for connection of cables to or from that equipment is considered as a port.

Ported number: A MSISDN that has undergone the porting process.

Ported subscriber: The subscriber of a ported number.

Porting process: A description of the transfer of a number between network operators.

Power control dynamic range: The difference between the maximum and the minimum total transmit output power for a specified reference condition (TS 25.104).

Predictive service: A service model which provides reliable performance, but allowing a specified variance in the measured performance criteria.

Prepay billing: Billing arrangement between customer and operator/service provider where the customer deposits an amount of money in advance, which is subsequently used to pay for service usage.

Postpay billing: Billing arrangement between customer and operator/service provider where the customer periodically receives a bill for service usage in the past period.

Proactive SIM: A SIM, which is capable of issuing commands to the Terminal. Part of SIM Application Toolkit.

Protocol: A formal set of procedures that are adopted to ensure communication between two or more functions within the within the same layer of a hierarchy of functions (source: ITU-T I.112).

Protocol data unit: In the reference model for OSI, a unit of data specified in an (N)-protocol layer and consisting of (N)-protocol control information and possibly (N)-user data (source: ITU-T X.200 / ISO-IEC 7498-1).

Public land mobile network: A telecommunications network providing mobile cellular services.

## Q

QoS profile: a QoS profile comprises a number of QoS parameters. A QoS profile is associated with each QoS session. The QoS profile defines the performance expectations placed on the bearer network.

QoS session: Lifetime of PDP context. The period between the opening and closing of a network connection whose characteristics are defined by a QoS profile. Multiple QoS sessions may exist, each with a different QoS profile.

Quality of Service: The collective effect of service performances which determine the degree of satisfaction of a user of a service. It is characterised by the combined aspects of performance factors applicable to all services, such as;

-	service operability performance;

-	service accessibility performance;

-	service retainability performance;

-	service integrity performance; and

-	other factors specific to each service.

## R

Radio access bearer: The service that the access stratum provides to the non-access stratum for transfer of user data between User Equipment and CN.

Radio Access Mode: Mode of the cell, FDD or TDD.

Radio Access Network Information Management: Functionality supporting the exchange of information, via the Core Network, between peer application entities located in a GERAN or in a UTRAN access network.

RAN sharing: Two or more CN operators share the same RAN, i.e. a RAN node (RNC or BSC) is connected to multiple CN nodes (SGSNs and MSC/VLRs) belonging to different CN operators.

Radio Access Network Application Part: Radio Network Signalling over the Iu.

Radio Access Network Operator: Operator that offers radio access to one or more core network operators.

Radio Access Technology:  Type of technology used for radio access, for instance E-UTRA, UTRA, GSM, CDMA2000 1xEV-DO (HRPD) or CDMA2000 1x (1xRTT).

Radio Bearer: The service provided by the Layer 2 for transfer of user data between User Equipment and UTRAN.

Radio communications equipment: Telecommunications equipment which includes one or more transmitters and/or receivers and/or parts thereof for use in a fixed, mobile or portable application. It can be operated with ancillary equipment but if so, is not dependent on it for basic functionality.

Radio digital unit: Equipment which contains base band and functionality for controlling Radio unit.

Radio equipment: Equipment which contains Radio digital unit and Radio unit.

Radio frame: A radio frame is a numbered time interval of 10 ms duration used for data transmission on the radio physical channel. A radio frame is divided into 15 time slots of 0.666 ms duration.  The unit of data that is mapped to a  radio frame (10 ms  time interval) may also be referred to as radio frame.

Radio interface: The "radio interface" is the tetherless interface between User Equipment and a UTRAN access point. This term encompasses all the functionality required to maintain such interfaces.

Radio link: A "radio link" is a logical association between single User Equipment and a single UTRAN access point. Its physical realisation comprises one or more radio bearer transmissions.

Radio link addition: The procedure where a new radio link is added to the active set.

Radio Link Control: A sublayer of radio interface layer 2 providing transparent, unacknowledged and  acknowledged data transfer service.

Radio link removal: The procedure where a radio link is removed from the active set.

Radio Link Set: A set of one or more Radio Links that has a common generation of Transmit Power Control (TPC) commands in the DL

Radio Network Controller: This equipment in the RNS is in charge of controlling the use and the integrity of the radio resources.

Radio Network Subsystem Application Part: Radio Network Signalling over the Iur.

Radio Network Subsystem: Either a full network or only the access part of a UTRAN offering the allocation and the release of specific radio resources to establish means of connection in between an UE and the UTRAN.
A Radio Network Subsystem is responsible for the resources and transmission/reception in a set of cells.

Radio Network Temporary Identifier: A Radio Network Temporary Identifier is a generic term of an identifier for a UE when an RRC connection exists. Following types of RNTI are defined: Cell RNTI (C-RNTI), Serving RNC RNTI (S-RNTI), UTRAN RNTI (U-RNTI) and GERAN RNTI (G-RNTI).

Radio Resource Control: A sublayer of radio interface Layer 3 existing in the control plane only which provides information transfer service to the non-access stratum. RRC is responsible for controlling the configuration of radio interface Layers 1 and 2.

Radio system: the selected 2nd  or 3rd generation radio access technology, eg UTRAN or GERAN.

Radio unit: Equipment which contains transmitter and receiver.

Rated Output Power:  For FDD BS, rated output power is the mean power level per carrier that the manufacturer has decared to be available at the antenna connector. For TDD BS rated output power is the mean power level per carrier over an active timeslot that the manufacturer has declared to be available at the antenna connector.

RE power control dynamic range: The difference between the power of a RE and the average RE power for a BS at maximum output power for a specified reference condition.

Real time: Time, typically in number of seconds, to perform the on-line mechanism used for fraud control and cost control.

Received Signal Code Power: Given only signal power is received, the average power of the received signal after despreading and combining.

Receiver Antenna Gain (dBi): The maximum gain of the receiver antenna in the horizontal plane (specified as dB relative to an isotropic radiator).

Receiver exclusion band: The receiver exclusion band is the band of frequencies over which no tests of radiated immunity of a receiver are made. The exclusion band for receivers is expressed relative to the base station receive band.

Receiver Noise Figure (dB): Receiver noise figure is the noise figure of the receiving system referenced to the receiver input.

Receiver Sensitivity (dBm): This is the signal level needed at the receiver input that just satisfies the required Eb/(No+Io).

Recipient network: The network which receives the number in the porting process. This network becomes the subscription network when the porting process is complete.

Record: A string of bytes within an EF handled as a single entity.

Record number: The number, which identifies a record within an EF.

Record pointer: The pointer, which addresses one record in an EF.

Reference bandwidth: The bandwidth in which an emission level is specified.

Reference configuration: A combination of functional groups and reference points that shows possible network arrangements (source: ITU-T I.112).

Reference point: A conceptual point at the conjunction of two non-overlapping functional groups (source: ITU-T I.112).

Regionally Provided Service: A service entitlement to only certain geographical part(s) of a PLMN, as controlled by the network operator.

Registration: This is the process of camping on a cell of the PLMN and doing any necessary LRs.

Registered PLMN (RPLMN): This is the PLMN on which the UE has performed a location registration successfully.

Registration Area: A (NAS) registration area is an area in which the UE may roam without a need to perform location registration, which is a NAS procedure.

Relay: Terminal devices capable of ODMA relay communications.

Relay/Seed Gateway: Relay or Seed that communicates with the UTRAN, in either TDD or FDD mode.

Relaylink: Relaylink is a communications link between two ODMA relay nodes.

Release 99: A particular version of the 3GPP System standards produced by the 3GPP project. Also: Release 4, Release 5, Release 6 etc..

Repeater:  A device that receives, amplifies and transmits the radiated or conducted RF carrier both in the down-link direction (from the base station to the mobile area) and in the up-link direction (from the mobile to the base station)

Requested QoS: a QoS profile is requested at the beginning of a QoS session. QoS modification requests are also possible during the lifetime of a QoS session.

Required Eb/(No+Io) (dB): The ratio between the received energy per information bit to the total effective noise and interference power density needed to satisfy the quality objectives.

Residual error rate: A parameter describing service accuracy. The frequency of lost SDUs, and of corrupted or duplicated network SDUs delivered at the user-network interface.

Retrieval service: An interactive service which provides the capability of accessing information stored in data base centres. The information will be sent to the user on demand only. The information is retrieved on an individual basis, i.e., the time at which an information sequence is to start is under the control of the user (source ITU-T I.113).

Roaming: The ability for a user to function in a serving network different from the home network. The serving network could be a shared network operated by two or more network operator.

Root directory: Obsolete term for Master File.

Root Relay: ODMA relay node where communications originate or terminate.

RRC Connection: A point-to-point bi-directional connection between RRC peer entities on the UE and the UTRAN sides, respectively. An UE has either zero or one RRC connection.

RRC filtered mean power: The mean power of a UTRA carrier as measured through a root raised cosine filter with roll-off factor a and a bandwidth equal to the chip rate of the radio access mode.

NOTE 1:	The RRC filtered mean power of a perfectly modulated UTRA signal is 0.246 dB lower than the mean power of the same signal.

## S

S1: interface between an eNB and an EPC, providing an interconnection point between the EUTRAN and the EPC. It is also considered as a reference point.

SDU error probability: The ratio of total incorrect service data units (SDUs) to total successfully transferred service data units plus incorrect service data units in a specified sample (source: ITU-T X.140).

NOTE:	the source document term "user information unit" has been replaced by the term "service data unit".

SDU loss probability: The ratio of total lost service data units (SDUs) to total transmitted service data units in a specified sample (source: ITU-T X.140).

NOTE:	the source document term "user information unit" has been replaced by the term "service data unit".

SDU misdelivery probability: The ratio of total misdelivered service data units (SDUs) to total service data units transferred between a specified source and destination user in a specified sample (source: ITU-T X.140).

NOTE:	the source document term "user information unit" has been replaced by the term "service data unit".

SDU transfer delay: The value of elapsed time between the start of transfer and successful transfer of a specified service data unit (SDU) (source: ITU-T X.140).

NOTE:	the source document term "user information unit" has been replaced by the term "service data unit".

SDU transfer rate: The total number of successfully transferred service data units (SDUs) in a transfer sample divided by the input/output time for that sample. The input/output time is the larger of the input time or the output time for the sample (source: ITU-T X.140).

NOTE:	the source document term "user information unit" has been replaced by the term "service data unit".

Seamless handover: "Seamless handover" is a handover without perceptible interruption of the radio connection.

Sector: A "sector" is a sub-area of a cell. All sectors within one cell are served by the same base station. A radio link within a sector can be identified by a single logical identification belonging to that sector.

Secured Packet: The information flow on top of which the level of required security has been applied. An Application Message is transformed with respect to a chosen Transport Layer and chosen level of security into one or more Secured Packets.

Security: The ability to prevent fraud as well as the protection of information availability, integrity and confidentiality.

Seed: Deployed ODMA relay node with or without a display/keypad.

Selected IP Traffic Offload (SIPTO): Offload of selected types of IP traffic (e.g. internet traffic) towards a defined IP network close to the UE's point of attachment to the access network. SIPTO is applicable to traffic offload for the macro-cellular access network and for the H(e)NB subsystem.

Selected PLMN: This is the PLMN that has been selected by the non-access stratum, either manually or automatically.

Service: a component of the portfolio of choices offered by service providers to a user, a functionality offered to a user.

Service-less UE: A UE that has only the Baseline capabilities.

Service Access Point: A conceptual point where a protocol layer offers access to its services to upper layer.

Service Area: The  Service Area is defined in the same way as the Service Area according to ITU-T Recommendation Q.1001 [4]. In contrast to the PLMN area it is not based on the coverage of a PLMN. Instead it is based on the area in which a fixed network user can call a mobile user without knowing his location. The Service Area can therefore change when the signalling system is being extended, for example.

Service attribute: A specified characteristic of a telecommunication service (source: ITU-T I.112).

NOTE:	the value(s) assigned to one or more service attributes may be used to distinguish that telecommunications service from others.

Service bit rate: The bit rate that is available to a user for the transfer of user information (source: ITU-T I.113).

Service Capabilities: Bearers defined by parameters, and/or mechanisms needed to realise services. These are within networks and under network control.

Service Capability Feature: Functionality offered by service capabilities that are accessible via the standardised application interface

Service Capability Server: Network functionality providing open interfaces towards  the functionality offered by 3GPP System service capabilities.

Service category or service class: A service offered to the users described by a set of performance parameters and their specified values, limits or ranges. The set of parameters provides a comprehensive description of the service capability.

Service Continuity: The uninterrupted user experience of a service that is using an active communication (e.g. an ongoing voice call) when a UE undergoes a radio access technology change  or a CS/PS domain change without, as far as possible, the user noticing the change.

NOTE: 	In particular Service Continuity encompasses the possibility that after a RAT / domain change the user experience is maintained by a different telecommunication service (e.g. tele- or bearer service) than before the RAT / domain change.

Service Control: The ability of the user, home environment or serving environment to determine what a particular service does, for a specific invocation of that service, within the limitations of that service.

Service Data Unit (SDU): In the reference model for OSI, an amount of information whose identity is preserved when transferred between peer (N+1)-layer entities and which is not interpreted by the supporting (N)-layer entities (source: ITU-T X.200 / ISO-IEC 7498-1).

Service delay: The time elapsed from the invocation of the service request, to the corresponding service request indication at the Service Receiver, indicating the arrival of application data.

Service Enabler: a capability which may be used, either by itself or in conjunction with other service enablers, to provide a service to the end user.

Service Execution Environment: A platform on which an application or programme is authorised to perform a number of functionalities; examples of service execution environments are the user equipment, integrated circuit card and a network platform or any other server.

Service Feature: Functionality that a 3GPP System shall offer to enable provision of services. Services, are made up of different service features.

Service Implementation Capabilities: Set of implementation capabilities, in each technical domain, required to enable a UE to support a set of UE Service Capabilities.

Service model: A general characterisation of services based upon a QoS paradigm, without specifying the actual performance targets.

Service Provider: A Service Provider is either a network operator or an other entity that provides services to a subscriber (e.g. a MVNO)

Service receiver: The entity which receives the service request indication primitive, containing the SDU.

Service relationship: The association between two or more entities engaged in the provision of services.

Service request: This is defined as being one invocation of the service through a service request primitive.

Service requester: The entity which requests the initiation of a GPRS operation, through a service request.

Service Specific Entities:  Entities dedicated to the provisioning of a given (set of) service(s). The fact that they are implemented or not in a given PLMN should have limited impact on all the other entities of the PLMN.

Service subscriber: Entity which subscribes to the General Packet Radio Service (GPRS) service.

Services (of a mobile cellular system): The set of unctions that the mobile cellular system can make available to the user.

Serving BSS: A role a BSS can take with respect to a specific connection between an MS and GERAN. There is one Serving BSS for each MS that has a connection to GERAN. The Serving BSS is in charge of the RRC connection between an MS and the GERAN. The Serving BSS terminates the Iu for this connection.

Serving Network: The serving network provides the user with access to the services of home environment.

Serving RNS: A role an RNS can take with respect to a specific connection between an UE and UTRAN. There is one Serving RNS for each UE that has a connection to UTRAN. The Serving RNS is in charge of the RRC connection between a UE and the UTRAN. The Serving RNS terminates the Iu for this connection.

Settlement: Payment of amounts resulting from the accounting process.

Shared Channel: A radio resource (transport channel or physical channel) that can be shared dynamically between several UEs.

Shared Network: When two or more network operator sharing network elements.

Short File Identifier (SFI): A 5-bit abbreviated name for a file in a directory on the UICC.

Short time: Time, typically in number of minutes, to perform the off-line mechanism used for accounting.

Signalling: The exchange of information specifically concerned with the establishment and control of connections, and with management, in a telecommunications network (source: ITU-T I.112).

Signalling connection: An acknowledged-mode link between the user equipment and the core network to transfer higher layer information between the entities in the non-access stratum.

Signalling link: Provides an acknowledged-mode link layer to transfer the UE-UTRAN signalling messages as well as UE - Core Network signalling messages  (using the signalling connection.

SIM application toolkit procedures: The portion of the communication protocol between the ME and the UICC that enables applications on the UICC to send commands to the ME.

SIM code: Code which when combined with the network and NS codes refers to a unique SIM. The code is provided by the digits 8 to 15 of the IMSI

(U)SIM code group: Combination of the (U)SIM code and the associated network subset and network codes (it is equivalent to the IMSI).

(U)SIM personalisation: Enables a user to personalise a ME so that it may only be used with particular (U)SIM(s).

Simultaneous use of services: The concurrent use of a circuit-mode service (voice or data) and packet-mode services (GPRS) by a single mobile station.

Soft Handover: Soft handover is a category of handover procedures where the radio links are added and abandoned in such manner that the UE always keeps at least one radio link to the UTRAN.

SP code: code which when combined with the network code refers to a unique SP.  The code is provided in the GID1 file on the SIM (see Annex A.1.) and is correspondingly stored on the ME.

SP code group: Combination of the SP code and the associated network code.

SP personalisation: Allows the service provider to personalise a ME so that it can only be used with that particular service provider's (U)SIMs.

Speed: A performance criterion that describes the time interval required to perform a function or the rate at which the function is performed. (The function may or may not be performed with the desired accuracy.) (source: ITU-T I.350).

SRNC Radio Network Temporary Identifier (S-RNTI): S-RNTI is UE identifier which is allocated by the Serving RNC and unique within this SRNC. It is allocated for all UEs having a RRC connection.  S-RNTI is reallocated always when the Serving RNC for the RRC connection is changed and deallocated when the RRC connection is released.

SRNS Relocation: The change of Iu instance and transfer of the SRNS role to another RNS.

Stratum: Grouping of protocols related to one aspect of the services provided by one or several domains.

Steering of Roaming: A technique whereby a roaming UE is encouraged to roam to a preferred VPLMN by the HPLMN.

Sub Network Management Functions: Set of functions that are related to a network model for a set of network elements constituting a clearly defined sub-network, which may include relations between the network elements. This model enables additional functions on the sub-network level (typically in the areas of network topology presentation, alarm correlation, service impact analysis and circuit provisioning).

Subscribed QoS: The network will not grant a QoS greater than the subscribed. The QoS profile subscription parameters are held in the HLR. An end user may have several QoS subscriptions. For security and the prevention of damage to the network, the end user cannot directly modify the QoS subscription profile data.

Subscriber: A Subscriber is an entity (associated with one or more users) that is engaged in a Subscription with a service provider. The subscriber is allowed to subscribe and unsubscribe services, to register a user or a list of users authorised to enjoy these services, and also to set the limits relative to the use that associated users make of these services.

Subscription: A subscription describes the commercial relationship between the subscriber and the service provider.

Subscription Management (SuM): set of capabilities that allow Operators, Service Providers, and indirectly subscribers, to provision, control, monitor the Subscription Profile.

Suitable Cell: This is a cell on which an UE may camp. It must satisfy certain conditions.

Supplementary service: A service which modifies or supplements a basic telecommunication service. Consequently, it cannot be offered to a user as a standalone service. It must be offered together with or in association with a basic telecommunication service. The same supplementary service may be common to a number of basic telecommunication services.

System Area: The  System Area is defined as the group of  PLMN areas accessible by  MSs. Interworking of several  PLMNs and interworking between  PLMNs and fixed network(s) permit  public land mobile communication services at international level.

## T

Teleaction service: A type of telecommunication service that uses short messages, requiring a low transmission rate, between the user and the network (source: ITU-T I.112).

Telecommunication port: Ports which are intended to be connected to telecommunication networks (e.g. public switched telecommunication networks, integrated services digital networks), local area networks (e.g. Ethernet, Token Ring) and similar networks.

Telecommunication service: What is offered by a PLMN operator or service provider to its customers in order to satisfy a specific telecommunication requirement. (source: ITU-T I.112). Telecommunication services are divided into two broad families: bearer services and teleservices (source: ITU-T I.210).

Teleservice: Is a type of telecommunication service that provides the complete capability, including terminal equipment functions, for communication between users according to standardised protocols and transmission capabilities established by agreement between operators.

Terminal: A device into which a UICC can be inserted and which is capable of providing access to 3GPP System services to users, either alone or in conjunction with a UICC.

Terminal Equipment (TE): Equipment that provides the functions necessary for the operation of the access protocols by the user. A functional group on the user side of a user-network interface (source: ITU-T I.112).

Test environment: A "test environment" is the combination of a test propagation environment and a deployment scenario, which together describe the parameters necessary to perform a detailed analysis of a radio transmission technology.

Text conversation: Real time transfer of text between users in at least two locations.

Text Telephony: An audiovisual conversation service providing bi-directional real time transfer of text and optionally audio between users in two locations. Audio may be transmitted alternating with text or simultaneously with text. (Source ITU-T F.703)

Transient phenomenon: Pertaining to or designating a phenomenon or a quantity which varies between two consecutive steady states during a time interval short compared with the time-scale of interest (IEC 60050-161 [6]).

Throughput: A parameter describing service speed. The number of data bits successfully  transferred in one direction between specified reference points per unit time (source: ITU-T I.113).

Toolkit applet: An application on the UICC that generates proactive commands to the ME.

Total Conversation: An audiovisual conversation service providing bi-directional symmetric real-time transfer of motion video, text and voice between users in two or more locations. (source ITU-T F.703)

Total power dynamic range: The difference between the maximum and the minimum total transmit output power for a specified reference condition (TS25.104).

Traffic channel: A "traffic channel" is a logical channel which carries user information.

Transit delay: A parameter describing service speed. The time difference between the instant at which the first bit of a protocol data unit (PDU) crosses one designated boundary (reference point), and the instant at which the last bit of the PDU crosses a second designated boundary (source: ITU-T I.113).

Transmission bandwidth: Bandwidth of an instantaneous transmission from a UE or BS, measured in Resource Block units.

Transmission bandwidth configuration: The highest transmission bandwidth allowed for uplink or downlink in a given channel bandwidth, measured in Resource Block units.

Transmission Time Interval: Transmission Time Interval is defined as the inter-arrival time of Transport Block Sets, i.e. the time it shall take to transmit a Transport Block Set.

Transmitter Antenna Gain (dBi): The maximum gain of the transmitter antenna in the horizontal plane (specified as dB relative to an isotropic radiator.

Transmitter exclusion band: The transmitter exclusion band is the band of frequencies over which no tests of radiated immunity of a transmitter are made. The exclusion band for transmitters is expressed relative to the carrier frequencies used (the carrier frequencies of the base stations activated transmitter(s).

Transmitter OFF period: The time period during which the BS transmitter is not allowed to transmit.

Transmitter ON period: The time period during which the BS transmitter is transmitting data and/or reference symbols, i.e. data subframes or DwPTS.

Transmitter transient period: The time period during which the transmitter is changing from the OFF period to the ON period or vice versa.

Transport Block: Transport Block is defined as the basic data unit exchanged between L1 and MAC.  An equivalent term for Transport Block is “MAC PDU”.

Transport Block Set: Transport Block Set is defined as a set of Transport Blocks that is exchanged between L1 and MAC at the same time instance using the same transport channel. An equivalent term for Transport Block Set is  “MAC PDU Set”.

Transport Block Set Size: Transport Block Set Size is defined as the number of bits  in a Transport Block Set.

Transport Block Size: Transport Block Size is defined as the size (number of bits) of a Transport Block.

Transport channel: The channels offered by the physical layer to Layer 2 for data transport between peer L1 entities are denoted as Transport Channels. Different types of transport channels are defined  by how and with which characteristics data is transferred on the physical layer, e.g. whether using dedicated or common physical channels.

Transport Format: A Transport Format is defined as a format offered by L1 to MAC for the delivery of a Transport Block Set during a Transmission Time Interval on a Transport Channel. The Transport Format constitutes of two parts – one dynamic part and one semi-static part.

Transport Format Combination: A Transport Format Combination is defined as the combination of currently valid Transport Formats on all Transport Channels of an UE, i.e. containing one Transport Format from each Transport Channel.

Transport Format Combination Set: A Transport Format Combination Set is defined as a set of Transport Format Combinations to be used by an UE.

Transport Format Combination Indicator (TFCI): A Transport Format Combination Indicator is a representation of the current Transport Format Combination.

Transport Format Identification  (TFI in UTRAN, TFIN in GERAN): A label for a specific Transport Format within a Transport Format Set.

Transport Format Set: A set of Transport Formats. For example, a variable rate DCH has a Transport Format Set (one Transport Format for each rate), whereas a fixed rate DCH has a single Transport Format.

## U

UE Service Capabilities: Capabilities that can be used either singly or in combination to deliver services to the user. The characteristic of UE Service Capabilities is that their logical function can be defined in a way that is independent of the implementation of the 3GPP System (although all UE Service Capabilities are of course constrained by the implementation of the 3GPP System). Examples: a data bearer of 144 kbps; a high quality speech teleservice; an IP teleservice; a capability to forward a speech call.

UICC: a physically secure device, an IC card (or 'smart card'), that can be inserted and removed from the terminal. It may contain one or more applications. One of the applications may be a USIM.

Universal Subscriber Identity Module (USIM): An application residing on the UICC used for accessing services provided by mobile networks, which the application is able to register on with the appropriate security.

Universal Terrestrial Radio Access Network (UTRAN): UTRAN is a conceptual term identifying that part of the network which consists of RNCs and Node Bs between Iu and Uu interfaces.

Usage Parameter Control (UPC): Set of actions taken by the network to monitor and control the offered traffic and the validity of the connection with respect to the traffic contract negotiated between the user and the network.

Uplink: An "uplink" is a unidirectional radio link for the transmission of signals from a UE  to a base station, from a Mobile Station to a mobile base station or from a mobile base station to a base station.

Uplink operating band: The part of the operating band designated for uplink.

Uplink Pilot Timeslot: Uplink part of the special subframe (for TDD operation)

Upper RF bandwidth edge: The frequency of the upper edge of the Base Station RF bandwidth, used as a frequency reference point for transmitter and receiver requirements

URA updating: URA updating is a family of procedures that updates the UTRAN registration area of a UE when a RRC connection exists and the position of the UE is known on URA level in the UTRAN.

User: An entity, not part of the 3GPP System , which uses 3GPP System services. Example: a person using a 3GPP System mobile station as a portable telephone.

User-network interface: The interface between the terminal equipment and a network termination at which interface the access protocols apply (source: ITU-T I.112).

User-user protocol: A protocol that is adopted between two or more users in order to ensure communication between them (source: ITU-T I.112).

User access or user network access: The means by which a user is connected to a telecommunication network in order to use the services and/or facilities of that network (source: ITU-T I.112).

User Equipment (UE): Allows a user access to network services. For the purpose of 3GPP specifications the interface between the UE and the network is the radio interface. A User Equipment can be subdivided into a number of domains, the domains being separated by reference points. Currently the User Equipment is subdivided into the UICC domain and the ME Domain. The ME Domain can further be subdivided into one or more Mobile Termination (MT) and Terminal Equipment (TE) components showing the connectivity between multiple functional groups.

In the context of Fixed Broadband Access to IMS, TISPAN defines the term UE in ETSI TR180 000 [5].

User Interface Profile: Contains information to present the personalised user interface within the capabilities of the terminal and serving network.

User Services Profile: Contains identification of subscriber services, their status and reference to service preferences.

UTRA Radio access mode: the selected UTRA radio access mode ie UTRA-FDD;UTRA-TDD.

UTRA-NTDD: Time Division Duplex UTRA access mode 1.28 Mcps option

UTRA-TDD:	Time Division Duplex UTRA Radio access mode (Includes UTRA-NTDD and UTRA-WTDD)

UTRA-WTDD: Time Division Duplex UTRA access mode 3.84 Mcps option

UTRAN access point: A conceptual point within the UTRAN performing radio transmission and reception. A UTRAN access point is associated with one specific cell, i.e. there exists one UTRAN access point for each cell. It is the UTRAN-side end point of a radio link.

UTRAN Registration Area: The UTRAN Registration Area is an area covered by a number of cells. The URA is only internally known in the UTRAN.

UTRAN Radio Network Temporary Identifier: The  U-RNTI is a unique UE identifier that consists of two parts, an SRNC identifier and a C-RNTI. U-RNTI is allocated to an UE having a RRC connection. It identifies the UE within UTRAN and is used as an UE identifier in cell update, URA update, RRC connection reestablishment and (UTRAN originated) paging messages and associated responses on the radio interface.

User Profile: Is the set of information necessary to provide a user with a consistent, personalised service environment, irrespective of the user’s location or the terminal used (within the limitations of the terminal and the serving network).

Uu: The Radio interface between UTRAN and the User Equipment.

## V

Value Added Service Provider: Provides services other than basic telecommunications service for which additional charges may be incurred.

Variable bit rate service: A type of telecommunication service characterised by a service bit rate specified by statistically expressed parameters which allow the bit rate to vary within defined limits (source: ITU-T I.113).

Virtual Home Environment: A concept for personal service environment portability across network boundaries and between terminals.

Virtual Machine: A software program that simulates a hypothetical computer central processing unit.  The programs executed by a virtual machine are represented as byte codes, which are primitive operations for this hypothetical computer.

Visited PLMN: This is a PLMN different from the HPLMN (if the EHPLMN list is not present or is empty) or different from an EHPLMN (if the EHPLMN list is present).

Visited PLMN of home country: This is a Visited PLMN where the MCC part of the PLMN identity is the same as the MCC of the IMSI.

## W

WTDD: Wide TDD – the 3.84 Mcps chip rate UTRA-TDD option.

WLAN UE: WLAN User Equipment: – a UE (equipped with UICC card including (U)SIM) utilized by a subscriber capable of accessing a WLAN network. A WLAN UE may include entities whose configuration, operation and software environment are not under the exclusive control of the 3GPP system operator, such as a laptop computer or PDA with a WLAN card, UICC card reader and suitable software applications.

## X

<void>

## Y

<void>

## Z

<void>

# 4	Abbreviations

## 0-9

1x RTT	CDMA2000 1x Radio Transmission Technology

2G	2nd Generation

3G	3rd Generation

3GPP	Third Generation Partnership Project

8-PSK	8-state Phase Shift Keying

## A

A-SGW	Access Signalling Gateway

A3	Authentication algorithm A3

A38	A single algorithm performing the functions of A3 and A8

A5/1	Encryption algorithm A5/1

A5/2	Encryption algorithm A5/2

A5/X	Encryption algorithm A5/0-7

A8	Ciphering key generating algorithm A8

AAL	ATM Adaptation Layer

AAL2	ATM Adaptation Layer type 2

AAL5	ATM Adaptation Layer type 5

AB 	Access Burst

AC	Access Class (C0 to C15)

Access Condition

Application Context

Authentication Centre

ACC	Automatic Congestion Control

ACELP	Algebraic Code Excited Linear Prediction

ACCH	Associated Control Channel

ACIR	Adjacent Channel Interference Ratio

ACK	Acknowledgement

ACL	APN Control List

ACLR	Adjacent Channel Leakage Power Ratio

ACM	Accumulated Call Meter

Address Complete Message

ACMmax	ACM (Accumulated Call Meter) maximal value

ACRR	Adjacent Channel Rejection Ratio

ACS	Adjacent Channel Selectivity

ACU	Antenna Combining Unit

ADC	Administration Centre

Analogue to Digital Converter

ADCH 	Associated Dedicated Channel

ADF	Application Dedicated File

ADM	Access condition to an EF which is under the control of the authority which creates this file

ADN	Abbreviated Dialling Numbers

ADPCM	Adaptive Differential Pulse Code Modulation

AE	Application Entity

AEC	Acoustic Echo Control

AEF	Additional Elementary Functions

AESA	ATM End System Address

AFC	Automatic Frequency Control

AGCH	Access Grant CHannel

Ai	Action indicator

AI	Acquisition Indicator

AICH	Acquisition Indicator Channel

AID	Application IDentifier

AIUR	Air Interface User Rate

AK	Anonymity Key

AKA	Authentication and Key Agreement

AKI	Asymmetric Key Index

ALCAP	Access Link Control Application Protocol

ALSI	Application Level Subscriber Identity

ALW	ALWays

AM	Acknowledged Mode

AMF	Authentication Management Field

AMN	Artificial Mains Network

AMR	Adaptive Multi Rate

AMR-WB	Adaptive Multi Rate Wide Band

AN	Access Network

ANP	Access Network Provider

AoC	Advice of Charge

AoCC	Advice of Charge Charging

AoCI	Advice of Charge Information

AP 	Access preamble

APDU	Application Protocol Data Unit

API	Application Programming Interface

APN	Access Point Name

ARFCN	Absolute Radio Frequency Channel Number

ARP	Address Resolution Protocol

ARQ	Automatic Repeat ReQuest

ARR	Access Rule Reference

AS	Access Stratum

ASC	Access Service Class

ASCI	Advanced Speech Call Items

ASE	Application Service Element

ASN.1	Abstract Syntax Notation One

AT command	ATtention Command

ATM	Asynchronous Transfer Mode

ATR	Answer To Reset

ATT (flag)	Attach

AU	Access Unit

AuC	Authentication Centre

AUT(H)	Authentication

AUTN	Authentication token

AWGN	Additive White Gaussian Noise

## B

B-ISDN	Broadband ISDN

BA	BCCH Allocation

BAIC	Barring of All Incoming Calls

BAOC	Barring of All Outgoing Calls

BC	Band Category

BCC	Base Transceiver Station (BTS) Colour Code

BCCH 	Broadcast Control Channel

BCD	Binary Coded Decimal

BCF	Base station Control Function

BCFE 	Broadcast Control Functional Entity

BCH	Broadcast Channel

BCIE	Bearer Capability Information Element

BDN	Barred Dialling Number

BER	Bit Error Ratio

Basic Encoding Rules (of ASN.1)

BFI	Bad Frame Indication

BG	Border Gateway

BGT	Block Guard Time

BI	all Barring of Incoming call

BIC	Baseline Implementation Capabilities

BIC-Roam	Barring of Incoming Calls when Roaming outside the home PLMN country

BID	Binding Identity

BLER	Block Error Ratio

Bm	Full-rate traffic channel

BMC	Broadcast/Multicast Control

BN	Bit Number

BO	all Barring of Outgoing call

BOC	Bell Operating Company

BOIC	Barring of Outgoing International Calls

BOIC-exHC	Barring of Outgoing International Calls except those directed to the Home PLMN Country

BPSK	Binary Phase Shift Keying

BS	Base Station

Basic Service (group)

Bearer Service

BSG	Basic Service Group

BSC	Base Station Controller

BSIC	Base transceiver Station Identity Code

BSIC-NCELL	BSIC of an adjacent cell

BSR	Buffer Status Report

BSS	Base Station Subsystem

BSSAP	Base Station Subsystem Application Part

BSSGP	Base Station Subsystem GPRS Protocol

BSSMAP	Base Station Subsystem Management Application Part

BSSOMAP	Base Station Subsystem Operation and Maintenance Application Part

BTFD	Blind Transport Format Detection

BTS   	Base Transceiver Station

BVC	BSS GPRS Protocol Virtual Connection

BVCI	BSS GPRS Protocol Virtual Connection Identifier

BW	Bandwidth

BWT	Block Waiting Time

## C

C	Conditional

C-	Control-

C/I	Carrier-to-Interference Power Ratio

CA	Carrier Aggregation

C-APDU	Command APDU

C-RNTI	Cell Radio Network Temporary Identity

C-TPDU	Command TPDU

CA	Capacity Allocation

Cell Allocation

Certification Authority

CAA	Capacity Allocation Acknowledgement

CAD	Card Acceptance Device

CAI	Charge Advice Information

CAMEL	Customised Application for Mobile network Enhanced Logic

CAP	CAMEL Application Part

CAZAC	Constant Amplitude Zero Auto-Correlation

CB	Cell Broadcast

CBC	Cell Broadcast Centre

Cipher Block Chaining

CBCH	Cell Broadcast CHannel

CBMI	Cell Broadcast Message Identifier

CBR	Constant Bit Rate

CBS	Cell Broadcast Service

CC	Call Control

Country Code

Cryptographic Checksum

Component Carrier

CC/PP	Composite Capability/Preference Profiles

CCBS	Completion of Calls to Busy Subscriber

CCCH	Common Control Channel

CCE	Control Channel Element

CCF	Call Control Function

CCH	Control Channel

CCI	Capability / Configuration Identifier

CCITT	Comité Consultatif International Télégraphique et Téléphonique (The International Telegraph and Telephone Consultative Committee)

CCK	Corporate Control Key

CCM	Certificate Configuration Message

Current Call Meter

CCO	Cell Change Order

CCP	Capability/Configuration Parameter

CCPCH	Common Control Physical Channel

Cct	Circuit

CCTrCH	Coded Composite Transport Channel

CD	Capacity Deallocation

Collision Detection

CDA	Capacity Deallocation Acknowledgement

CDCH	Control-plane Dedicated CHannel

CDMA	Code Division Multiple Access

CDN	Coupling/Decoupling Network

CDR	Charging Data Record

CDUR	Chargeable DURation

CED	called station identifier

CEIR	Central Equipment Identity Register

CEND	end of charge point

CEPT	Conférence des administrations Européennes des Postes et Telecommunications

CF	Conversion Facility

all Call Forwarding services

CFB	Call Forwarding on mobile subscriber Busy

CFN	Connection Frame Number

CFNRc	Call Forwarding on mobile subscriber  Not Reachable

CFNRy	Call Forwarding on No Reply

CFU	Call Forwarding Unconditional

CGI	Common Gateway Interface

Cell Global Identifier

CHAP	Challenge Handshake Authentication Protocol

CHP	CHarging Point

CHV	Card Holder Verification information

CI	Cell Identity

CUG index

CID	Cell-ID (positioning method)

CIM	Common Information Model

CIR	Carrier to Interference Ratio

CK	Cipher Key

CKSN	Ciphering Key Sequence Number

CLA	CLAss

CLI	Calling Line Identity

CLIP	Calling Line Identification Presentation

CLIR	Calling Line Identification Restriction

CLK	Clock

CM	Connection Management

CMAS	Commercial Mobile Alert Service

CMC	Connection Mobility Control

CMD	Command

CMIP	Common Management Information Protocol

CMISE	Common Management Information Service

CMM	Channel Mode Modify

CN	Core Network

Comfort Noise

CNAP	Calling Name Presentation

CNG	Calling Tone

CNL	Co-operative Network List

CNTR	Counter

CLNP	Connectionless network protocol

CLNS	Connectionless network service

COLI	COnnected Line Identity

COLP	COnnected Line identification Presentation

COLR	COnnected Line identification Restriction

COM	COMplete

CONNACK	Connect Acknowledgement

CONS	Connection-oriented network service

CORBA	Common Object Request Broker Architecture

CP	Cyclic prefix

CP-Admin	Certificate Present (in the MExE SIM)-Administrator

CP-TP	Certificate Present (in the MExE SIM)-Third Party

CPBCCH	COMPACT Packet BCCH

CPICH	Common Pilot Channel

CPCH	Common Packet Channel

CPCS	Common Part Convergence Sublayer

CPS	Common Part Sublayer

CPU	Central Processing Unit

C-plane	Control Plane

C/R	Command/Response field bit

CQI	Channel Quality Indicator

CRC	Cyclic Redundancy Check

CRE	Call Ree-establishment procedure

CRNC	Controlling Radio Network Controller

CS-GW	Circuit Switched Gateway

CS	Circuit Switched

Coding Scheme

CSCF	Call Server Control Function

CSD	Circuit Switched Data

CSE	Camel Service Environment

CSG	Closed Subscriber Group

CSGID	Closed Subscriber Group Identity

CSI	Channel State Information

CSPDN	Circuit Switched Public Data Network

CT	Call Transfer supplementary service

Channel Tester

Channel Type

CTCH	Common Traffic Channel

CTDMA	Code Time Division Multiple Access

CTFC	Calculated Transport Format Combination

CTM	Cellular Text telephone Modem

CTR	Common Technical Regulation

CTS	Cordless Telephony System

CUG	Closed User Group

CW	Call Waiting

Continuous Wave (unmodulated signal)

CWI	Character Waiting Integer

CWT	Character Waiting Time

C-RNTI	Cell RNTI

## D

DAC	Digital to Analog Converter

DAD	Destination ADress

DAM	DECT Authentication Module

DB	Dummy Burst

DC	Dedicated Control (SAP)

Direct Current

DCA	Dynamic Channel Allocation

DCCH	Dedicated Control Channel

DCE	Data Circuit terminating Equipment

DCF	Data Communication Function

DCH	Dedicated Channel

DCI	Downlink Control Information

DCK	Depersonalisation Control Key

DCN	Data Communication Network

DCS	Data Coding Scheme

DCS1800	Digital Cellular Network at 1800MHz

DC-HSDPA	 Dual Cell HSDPA

DDI	Direct Dial In

DECT	Digital Enhanced Cordless Telecommunications

DET	Detach

DES	Data Encryption Standard

DF	Dedicated File

DFT	Discrete Fourier Transformation

DHCP	Dynamic Host Configuration Protocol

DHO	Diversity Handover

diff-serv	Differentiated services

DISC	Disconnect

DL	Data Layer

Downlink (Forward Link)

DLCI	Data Link Connection Identifier

DLD	Data Link Discriminator

DL-SCH	Downlink Shared channel

Dm	Control channel (ISDN terminology applied to mobile service)

DMR	Digital Mobile Radio

DMTF	Distributed Management Task Force

DN	Destination Network

DNIC	Data Network Identifier

DNS	Directory Name Service

DO	Data Object

DP	Dial/Dialled Pulse

DPCCH	Dedicated Physical Control Channel

DPCH	Dedicated Physical Channel

DPDCH	Dedicated Physical Data Channel

DRAC	Dynamic Resource Allocation Control

DRB	Data Radio Bearer

DRNC	Drift Radio Network Controller

DRNS	Drift RNS

DRX	Discontinuous Reception

DS-CDMA	Direct-Sequence Code Division Multiple Access

DSAC	Domain Specific Access Control

DSCH	Downlink Shared Channel

DSE	Data Switching Exchange

DSI	Digital Speech Interpolation

DSS1	Digital Subscriber Signalling No1

DTAP	Direct Transfer Application Part

DTCH	Dedicated Traffic Channel

DTE	Data Terminal Equipment

DTMF	Dual Tone Multiple Frequency

DTT	Digital Terrestrial Television

DTX	Discontinuous Transmission

DUT	Device Under Test

DwPTS	Downlink Pilot Timeslot

## E

E-CID	Enhanced Cell-ID (positioning method)

E-GGSN	Enhanced GGSN

E-HLR	Enhanced HLR

E-RAB	E-UTRAN Radio Access Bearer

E-SMLC	Enhanced Serving Mobile Location Centre

E-TM	E-UTRA Test Model

EA	External Alarms

EARFCN	E-UTRA Absolute Radio Frequency Channel Number

EBSG	Elementary Basic Service Group

ECB	Electronic Code-book

ECC	Emergency Call Code

Elliptic Curve Cryptography

ECEF	Earth Centred, Earth Fixed

ECGI	E-UTRAN Cell Global Identifier

ECI	Earth-Centered-Inertial

ECM	Error Correction Mode (facsimile)

EPS Connection Management

Ec/No	Ratio of energy per modulating bit to the noise spectral density

ECSD	Enhanced CSD

ECT	Explicit Call Transfer supplementary service

ECTRA	European Committee of Telecommunications Regulatory Affairs

EDC	Error Detection Code byte

EDGE	Enhanced Data rates for GSM Evolution

EEL	Electric Echo Loss

EF	Elementary File (on the UICC)

EFR	Enhanced Full Rate

EFS	Error free seconds

EGPRS	Enhanced GPRS

EHPLMN	Equivalent Home PLMN

EIR	Equipment Identity Centre

Equipment Identity Register

EIRP	Equivalent Isotropic Radiated Power

EL	Echo Loss

EF	Elementary File

EM	Element Manager

EMC	ElectroMagnetic Compatibility

eMLPP	enhanced Multi-Level Precedence and Pre-emption

EMMI	Electrical Man Machine Interface

eNB	E-UTRAN Node B

evolved Node B

EP	Elementary Procedure

EPA	Extended Pedestrian A model

EPC	Enhanced Power Control

Evolved Packet Core

EPRE	Energy Per Resource Element

E-UTRA	Evolved UTRA

Evolved Universal Terrestrial Radio Access

E-UTRAN	Evolved UTRAN

Evolved Universal Terrestrial Radio Access Network

EPS	Evolved Packet System

EPCCH	Enhanced Power Control Channel

EPROM	Erasable Programmable Read Only Memory

ERP	Ear Reference Point

Equivalent Radiated Power

ERR	Error

ESD	Electrostatic discharge

ETNS	European Telecommunications Numbering Space

ETR	ETSI Technical Report

ETS	European Telecommunication Standard

ETSI	European Telecommunications Standards Institute

etu	elementary time unit

ETU	Extended Typical Urban model

ETWS	Earthquake and Tsunami Warning System

EUI	End-User Identity

EVA	Extended Vehicular A model

EVM	Error Vector Magnitude

## F

FA	Full Allocation

Fax Adaptor

FAC	Final Assembly Code

FACCH	Fast Associated Control CHannel

FACCH/F	Fast Associated Control Channel/Full rate

FACCH/H	Fast Associated Control Channel/Half rate

FACH 	Forward Access Channel

FAUSCH	Fast Uplink Signalling Channel

FAX	Facsimile

FB	Frequency correction Burst

FBI	Feedback Information

FCC	Federal Communications Commission

FCCH	Frequency Correction CHannel

FCI	File Control Information

FCP	File Control Parameter

FCS	Frame Check Sequence

FDD	Frequency Division Duplex

FDM	Frequency Division Multiplex

FDMA	Frequency Division Multiple Access

FDN	Fixed Dialling Number

FDR	False transmit format Detection Ratio

FEC	Forward Error Correction

FER	Frame Erasure Rate, Frame Error Rate

FFS	For Further Study

FFT	Fast Fourier Transformation

FH	Frequency Hopping

FLO	Flexible Layer One

FM	Fault Management

FMC	Fixed Mobile Convergence

FN	Frame Number

FNUR	Fixed Network User Rate

FP	Frame Protocol

FPLMN	Forbidden PLMN

FR	Full Rate

FRC	Fixed Reference Channel

FTAM	File Transfer Access and Management

ftn	forwarded-to number

## G

G-RNTI	GERAN Radio Network Temporary Identity

GAGAN	GPS Aided Geo Augmented Navigation

GBR	Guaranteed Bit Rate

GC	General Control (SAP)

GCR	Group Call Register

GERAN	GSM EDGE Radio Access Network

GGSN	Gateway GPRS Support Node

GID1	Group Identifier (level 1)

GID2	Group Identifier (level 2)

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System)

GMLC	Gateway Mobile Location Centre

GMM	GPRS Mobility Management

GMSC	Gateway MSC

GMSK	Gaussian Minimum Shift Keying

GP	Guard Period

GPA	GSM PLMN Area

GPRS	General Packet Radio Service

GPS	Global Positioning System

GRA	GERAN Registration Area

GSA	GSM System Area

GSIM	GSM Service Identity Module

GSM	Global System for Mobile communications

GSN	GPRS Support Nodes

GT	Global Title

GTP	GPRS Tunneling Protocol

GTP-U	GPRS Tunnelling Protocol for User Plane

GTT	Global Text Telephony

GUMMEI	Globally Unique MME Identifier

GUP	3GPP Generic User Profile

## H

H-CSCF	Home CSCF

HANDO	Handover

HARQ	Hybrid ARQ, Hybrid Automatic Repeat Request

HCS	Hierarchical Cell Structure

HDLC	High Level Data Link Control

HE	Home Environment

HE-VASP	Home Environment Value Added Service Provider

HF	Human Factors

HFN	HyperFrame Number

HHO	Hard Handover

HLC	High Layer Compatibility

HLR	Home Location Register

HN	Home Network

HO	Handover

HOLD	Call hold

HPLMN	Home Public Land Mobile Network

HPS	Handover Path Switching

HPU	Hand Portable Unit

HR	Half Rate

HRPD	CDMA2000 High Rate Packet Data

HRR	Handover Resource Reservation

HSCSD	High Speed Circuit Switched Data

HSDPA	High Speed Downlink Packet Access

HSN	Hopping Sequence Number

HSPA	High Speed Packet Access

HSS	Home Subscriber Server

HSUPA	High Speed Uplink Packet Access

HTTP	Hyper Text Transfer Protocol

HTTPS	Hyper Text Transfer Protocol Secure (https is http/1.1 over SSL, i.e. port 443)

HU	Home Units

## I

I-Block	Information Block

I-ETS	Interim European Telecommunications Standard

I/O	Input/Output

I	Information frames (RLP)

IA	Incoming Access (closed user group SS)

IAM	Initial Address Message

IC	Integrated Circuit

Interlock Code (CUG SS)

IC(pref)	Interlock Code of the preferential CUG

ICB	Incoming Calls Barred (within the CUG)

ICC	Integrated Circuit Card

ICCID	Integrated Circuit Card Identification

ICD	Interface Control Document

ICGW	Incoming Call Gateway

ICI 	Incoming Call Information

ICIC	Inter-Cell Interference Coordination

ICM	In-Call Modification

ICMP	Internet Control Message Protocol

ICS	In-Channel Selectivity

ICT	Incoming Call Timer

ID	Identifier

IDFT	Inverse Discrete Fourier Transform

IDL	Interface Definition Language

IDN	Integrated Digital Network

IDNNS	Intra Domain NAS Node Selector

IE	Information Element

IEC	International Electrotechnical Commission

IED	Information Element Data

IEI	Information Element Identifier

IEIDL	Information Element Identifier Data Length

IETF	Internet Engineering Task Force

IF	Infrastructure

IFD	Interface Device

IFOM	IP FlOw Mobility

IFS	Information Field Sizes

IFSC	Information Field Size for the UICC

IFSD	Information Field Size for the Terminal

IHOSS	Internet Hosted Octet Stream Service

IIOP	Internet Inter-ORB Protocol

IK	Integrity key

IM	Intermodulation

IP Multimedia

IMA	Inverse Multiplexing on ATM

IMC	IMS Credentials

IMEI	International Mobile Equipment Identity

IMGI	International mobile group identity

IMPI	IP Multimedia Private Identity

IMPU	IP Multimedia PUblic identity

IMS	IP Multimedia Subsystem

IMSI	International Mobile Subscriber Identity

IMT-2000 	International Mobile Telecommunications 2000

IMUN	International Mobile User Number

IN	Intelligent Network

Interrogating Node

INAP	Intelligent Network Application Part

INF	INFormation field

IP	Internet Protocol

IP-CAN	IP-Connectivity Access Network

IP-M	IP Multicast

IPv4	Internet Protocol Version 4

IPv6	Internet Protocol Version 6

IR	Infrared

IRP	Integration Reference Point

IS	Interface Specification

ISC	International Switching Centre

ISCP	Interference Signal Code Power

ISDN	Integrated Services Digital Network

ISIM	IM Services Identity Module

ISO	International Organisation for Standardisation

ISP	Internet Service Provider

ISUP	ISDN User Part

ITC	Information Transfer Capability

ITU	International Telecommunication Union

ITU-R	Radiocommunication Sector of the ITU

IUI	International USIM Identifier

IUT	Implementation Under Test

IWF	InterWorking Function

I-WLAN	Interworking WLAN

IWMSC	InterWorking MSC

IWU	Inter Working Unit

## J

JAR file	Java Archive File

JCRE	Java Card™ Run Time Environment

JD	Joint Detection

JNDI	Java Naming Directory Interface

JP	Joint Predistortion

JPEG	Joint Photographic Experts Group

JTAPI	Java Telephony Application Programming Interface

JVM	Java™ Virtual Machine

## K

k	Windows size

K	Constraint length of the convolutional code

USIM Individual key

kbps	kilo-bits per second

Kc	Ciphering key

Ki	Individual subscriber authentication key

KSI	Key Set Identifier

ksps	kilo-symbols per second

## L

L1	Layer 1 (physical layer)

L2	Layer 2 (data link layer)

L2ML	Layer 2 Management Link

L2R	Layer 2 Relay

L2R BOP	L2R Bit Orientated Protocol

L2R COP	L2R Character Orientated Protocol

L3	Layer 3 (network layer)

LA	Location Area

LAC	Link Access Control

Location Area Code

LAI	Location Area Identity

LAN	Local Area Network

LAPB	Link Access Protocol Balanced

LAPDm	Link Access Protocol on the Dm channel

LATA	Local Access and Transport Area

LAU	Location Area Update

LB	Load Balancing

LCD	Low Constrained Delay

LCG	Logical Channel Group

LCN	Local Communication Network

LCP	Link Control Protocol

LCR	Low Chip Rate

LCS	Location Services

LCSC	LCS Client

LCSS	LCS Server

LE	Local Exchange

LEN	LENgth

LI	Language Indication

Length Indicator

Line Identity

LIPA	Local IP Access

LLC	Logical Link Control

Low Layer Compatibility

Lm	Traffic channel with capacity lower than a Bm

LMSI	Local Mobile Station Identity

LMU	Location Measurement Unit

LN	Logical Name

LNA	Low Noise Amplifier

LND	Last Number Dialled

LNS	L2TP Network Server

LPLMN	Local PLMN

LPP	LTE Positioning Protocol

LPPa	LTE Positioning Protocol Annex

LR	Location Register

Location Registration

LSA	Localised Service Area

LSB	Least Significant Bit

LSTR	Listener SideTone Rating

LTE	Local Terminal Emulator

Long Term Evolution

LTZ	Local Time Zone

LU	Local Units

Location Update

LV	Length and Value

## M

M	Mandatory

M	Mandatory

MA	Mobile Allocation

Multiple Access

MAC	Medium Access Control (protocol layering context)

Message authentication code (encryption context)

MAC-A	MAC used for authentication and key agreement (TSG T WG3 context)

MAC-I	MAC used for data integrity of signalling messages (TSG T WG3 context)

MACN	Mobile Allocation Channel Number

MAF	Mobile Additional Function

MAH	Mobile Access Hunting supplementary service

MAHO	Mobile Assisted Handover

MAI	Mobile Allocation Index

MAIO	Mobile Allocation Index Offset

MAP	Mobile Application Part

MBMS	Multimedia Broadcast and Multicast Service

MBSFN

Multimedia Broadcast multicast service Single Frequency Network

MCC	Mobile Country Code

MCCH	Multicast Control Channel

MCE	Multi-cell/multicast Coordination Entity

MCH	Multicast channel

MCI	Malicious Call Identification supplementary service

MCML	Multi-Class Multi-Link PPP

Mcps	Mega-chips per second

MCS	Modulation and Coding Scheme

MCU	Media Control Unit

MD	Mediation Device

MDL	(mobile) Management (entity) - Data Link (layer)

MDS	Multimedia Distribution Service

MDT	Minimization of Drive Tests

ME	Maintenance Entity

Mobile Equipment

MEF	Maintenance Entity Function

MEHO 	Mobile evaluated handover

MER 	Message Error Ratio

MExE	Mobile Execution Environment

MF	Master File

MultiFrame

MGCF	Media Gateway Control Function

MGCP	Media Gateway Control Part

MGT	Mobile Global Title

MGW	Media GateWay

MHEG 	Multimedia and Hypermedia Information Coding Expert Group

MHS	Message Handling System

MIB	Management Information Base

Master Information Block

MIC	Mobile Interface Controller

MIM	Management Information Model

MIMO	Multiple Input Multiple Output

MIP	Mobile IP

MIPS	Million Instructions Per Second

MLC	Mobile Location Centre

MM	Man Machine

Mobility Management

Multimedia

MME	Mobile Management Entity

MMI	Man Machine Interface

MNC	Mobile Network Code

MNP	Mobile Number Portability

MO	Mobile Originated

MO-LR	Mobile Originating Location Request

MO-SMS	Mobile Originated Short Message Service

MOHO	Mobile Originated Handover

MOS	Mean Opinion Score

MoU	Memorandum of Understanding

MP	Multi-link PPP

MPEG	Moving Pictures Experts Group

MPH	(mobile) Management (entity) - PHysical (layer) [primitive]

MPTY	MultiParTY

MRF	Media Resource Function

MRP	Mouth Reference Point

MS	Mobile Station

MSA	MCH Subframe Allocation

MSB	Most Significant Bit

MSC	Mobile Switching Centre

MSCM	Mobile Station Class Mark

MSCU	Mobile Station Control Unit

MSD	Maximum Sensitivity Degradation

MSE	MExE Service Environment

MSID	Mobile Station Identifier

MSD	Maximum Sensitivity Degradation

MSI	MCH Scheduling Information

MSIN	Mobile Station Identification Number

MSISDN	Mobile Subscriber ISDN Number

MSP	Multiple Subscriber Profile

MSR	Multi-Standard Radio

MSRN	Mobile Station Roaming Number

MT	Mobile Terminated

Mobile Termination

MTCH	Multicast Traffic Channel

MT-LR	Mobile Terminating Location Request

MT-SMS	Mobile Terminated Short Message Service

MTM	Mobile-To-Mobile (call)

MTP	Message Transfer Part

MTP3-B	Message Transfer Part level 3

MTU	Maximum Transfer Unit

MU	Mark Up

MUI	Mobile User Identifier

MUMS	Multi User Mobile Station

MVNO	Mobile Virtual Network Operator

## N

NACC	Network Assisted Cell Change

NACK	Negative Acknowledgement

NAD	Node Address byte

NAI	Network Access Identifier

NAS	Non-Access Stratum

NBAP	Node B Application Part

NB	Normal Burst

NCELL	Neighbouring (of current serving) Cell

NBAP	Node B Application Part

NBIN	A parameter in the hopping sequence

NCC	Network (PLMN) Colour Code

NCH	Notification CHannel

NCK	Network Control Key

NCP	Network Control Protocol

NDC	National Destination Code

NDUB	Network Determined User Busy

NE	Network Element

NEF	Network Element Function

NEHO 	Network evaluated handover

NET	NETwork

Norme Europeenne de Télécommunications

NEV	NEVer

NF	Network Function

NI-LR	Network Induced Location Request

NIC	Network Independent Clocking

NITZ	Network Identity and Time Zone

NM	Network Manager

NMC	Network Management Centre

NMR	Network Measurement Results

NMO	Network Mode of Operation

NMS	Network Management Subsystem

NMSI	National Mobile Station Identifier

NNI	Network-Node Interface

NO	Network Operator

NP	Network Performance

NPA	Numbering Plan Area

NPI	Numbering Plan Identifier

NRI	Network Resource Identifier

NRM	Network Resource Model

NRT	Non-Real Time

NSAP	Network Service Access Point

NSAPI	Network Service Access Point Identifier

NSCK	Network Subset Control Key

NSDU	Network service data unit

NSS	Network Sub System

Nt	Notification (SAP)

NT	Network Termination

Non Transparent

NTAAB	New Type Approval Advisory Board

NTDD	Narrow-band Time Division Duplexing

NUA	Network User Access

NUI	National User / USIM Identifier

Network User Identification

NUP	National User Part (SS7)

NW	Network

## O

O	Optional

O&M	Operations & Maintenance

OA	Outgoing Access (CUG SS)

OACSU	Off-Air-Call-Set-Up

OCB	Outgoing Calls Barred within the CUG

OCCCH	ODMA Common Control Channel

OCF	Open Card Framework

OCI	Outgoing Call Information

OCNG	OFDMA Channel Noise Generator

OCNS	Orthogonal Channel Noise Simulator

OCT	Outgoing Call Timer

OD	Optional for operators to implement for their aim

ODB	Operator Determined Barring

ODCCH	ODMA Dedicated Control Channel

ODCH	ODMA Dedicated Channel

OLR	Overall Loudness Rating

ODMA	Opportunity Driven Multiple Access

ODTCH	ODMA Dedicated Traffic Channel

OID	Object Identifier

OFDM	Orthogonal Frequency Division Multiplex

Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OFM	Operational Feature Monitor

OMC	Operation and Maintenance Centre

OML	Operations and Maintenance Link

OOB	Out-of-band

OPLMN	Operator Controlled PLMN (Selector List)

OR	Optimal Routeing

ORACH	ODMA Random Access CHannel

ORLCF	Optimal Routeing for Late Call Forwarding

OS	Operations System

OSA	Open Service Access

OSI	Open System Interconnection

OSI RM	OSI Reference Model

OSP	Octet Stream Protocol

OSP:IHOSS	Octet Stream Protocol for Internet Hosted Octet Stream Service

OTA	Over-The-Air

OTDOA	Observed Time Difference Of Arrival (positioning method)

OTP	One Time Password

OVSF	Orthogonal Variable Spreading Factor

## P

P-CCPCH	Primary Common Control Physical Channel

P-CPIH	Primary Common Pilot Channel

P-RNTI	Paging RNTI

P-TMSI	Packet TMSI

PA	Power Amplifier

PAPR	Peak-to-Average Power Ratio

PABX	Private Automatic Branch eXchange

PACCH	Packet Associated Control Channel

PAD	Packet Assember/Disassembler

PAGCH	Packet Access Grant Channel

PAP	Password Authentication Protocol

PAR	Peak to Average Ratio

PB	Pass Band

PBID	PhoneBook IDentifier

PBCCH	Packet Broadcast Control Channel

PBCH	Physical Broadcast Channel

PBP	Paging Block Periodicity

PBX	Private Branch eXchange

PC	Power Control

Personal Computer

PCB	Protocol Control Byte

PCCC	Parallel Concatenated Convolutional Code

PCCCH	Packet Common Control Channel

PCCH	Paging Control Channel

PCDE	Peak Code Domain Error

PCell	Primary Cell

PCFICH	Physical control format indicator channel

PCG	Project Co-ordination Group

PCH 	Paging Channel

PCK	Personalisation Control Key

PCM	Pulse Code Modulation

PCMCIA	Personal Computer Memory Card International Association

PCPCH	Physical Common Packet Channel

PCS 	Personal Communication System

PCU	Packet Control Unit

PD	Protocol Discriminator

Public Data

PDCCH	Physical Downlink Control Channel

PDCP	Packet Data Convergence Protocol

PDCH	Packet Data Channel

PDH	Plesiochronous Digital Hierarchy

PDN	Public Data Network

Packet Data Network

PDP	Packet Data Protocol

PDSCH	Physical Downlink Shared Channel

PDTCH	Packet Data Traffic Channel

PDU	Protocol Data Unit

PG	Processing Gain

PH	Packet Handler

PHysical (layer)

PHF	Packet Handler Function

PHI	Packet Handler Interface

PHICH	Physical hybrid-ARQ indicator channel

PHS 	Personal Handyphone System

PHY	Physical layer

PhyCH	Physical Channel

PI	Page Indicator

Presentation Indicator

PICH	Page Indicator Channel

PICS	Protocol Implementation Conformance Statement

PID	Packet Identification

PIN	Personal Identification Number

PIXT	Protocol Implementation eXtra information for Testing

PKCS	Public-Key Cryptography Standards

PL	Preferred Languages

PLMN	Public Land Mobile Network

PMCH	Physical Multicast Channel

PMD	Physical Media Dependent

PMI	Precoding Matrix Indicator

PN	Pseudo Noise

PNE	Présentation des Normes Européennes

PNP	Private Numbering Plan

POI	Point Of Interconnection (with PSTN)

PoR	Proof of Receipt

POTS	Plain Old Telephony Service

PP	Point-to-Point

PPCH	Packet Paging Channel

PPE	Primative Procedure Entity

PPF	Paging Proceed Flag

PPM	Parts Per Million

PPP	Point-to-Point Protocol

PPS	Protocol and Parameter Select (response to the ATR)

PRACH	Physical Random Access Channel

Packet Random Access Channel

PRB	Physical Resource Block

Pref CUG	Preferential CUG

PRS	Positioning Reference Signal

PS	Packet Switched

Location Probability

PSC	Primary Synchronisation Code

Packet Scheduling

PSCH	Physical Shared Channel

PSE	Personal Service Environment

PSPDN	Packet Switched Public Data Network

PSTN	Public Switched Telephone Network

PTCCH	Packet Timing advance Control Channel

PTM	Point-to-Multipoint

PTM-G	PTM Group Call

PTM-M	PTM Multicast

PTP	Point to point

PU	Payload Unit

PUCCH	Physical Uplink Control Channel

PUCT	Price per Unit Currency Table

PUK	PIN Unblocking Key

PUSCH	Physical Uplink Shared Channel

PVC	Permanent Virtual Circuit

PW	Pass Word

PWS	Public Warning System

## Q

QA	Q (Interface) - Adapter

QAF	Q - Adapter Function

QAM	Quadrature Amplitude Modulation

QCI	QoS Class Identifier

QoS	Quality of Service

QPSK	Quadrature (Quaternary) Phase Shift Keying

QZSS	Quasi-Zenith Satellite System

## R

R	Value of Reduction of the MS transmitted RF power relative to the maximum allowed output power of the highest  power class of MS (A)

R-APDU	Response APDU

R-Block	Receive-ready Block

R-PDCCH	Relay Physical Downlink Control Channel

R-SGW	Roaming Signalling Gateway

R-TPDU	Response TPDU

R99	Release 1999

RA	Routing Area

Random mode request information field

RA-RNTI	Random Access RNTI

RAB	Radio Access Bearer

Random Access Burst

RAC	Routing Area Code

RACH	Random Access Channel

RADIUS	Remote Authentication Dial In User Service

RAI	Routing Area Identity

RAN	Radio Access Network

RANAP	Radio Access Network Application Part

RAND	RANDom number (used for authentication)

RAT	Radio Access Technology

RAU	Routing Area Update

RB	Radio Bearer

RBC	Radio Bearer Control

RBER	Residual Bit Error Ratio

RDF	Resource Description Format

RDI	Restricted Digital Information

RE	Resource Element

REC	RECommendation

REG	Resource Element Group

REJ	REJect(ion)

REL	RELease

Rel-4	Release 4

Rel-5	Release 5

REQ	REQuest

RES	user RESponse

64-bit signed RESponse that is the output of the function f2 in a 3G AKA

RET	Remote Electrical Tilting

RETAP	Remote Electrical Tilting Application Part

RF	Radio Frequency

RFC	Request For Comments

Radio Frequency Channel

RFCH	Radio Frequency CHannel

RFE	Routing Functional Identity

RFN	Reduced TDMA Frame Number

RFU	Reserved for Future Use

RI	Rank Indication

RIM	RAN Information Management

RL	Radio Link

RLC	Radio Link Control

RLCP	Radio Link Control Protocol

RLP	Radio Link Protocol

RLR	Receiver Loudness Rating

RLS	Radio Link Set

RMS	Root Mean Square (value)

RN	Relay Node

RNC	Radio Network Controller

RNL	Radio Network Layer

RNS	Radio Network Subsystem

RNSAP	Radio Network Subsystem Application Part

RNTABLE	Table of 128 integers in the hopping sequence

RNTI	Radio Network Temporary Identity

ROHC	Robust Header Compression

RPLMN	Registered Public Land Mobile Network

RPOA	Recognised Private Operating Agency

RR	Radio Resources

RRC	Radio Resource Control

RRM	Radio Resource Management

RS	Reference Symbol

RSA	Algorithm invented by Rivest, Adleman and Shamir

RSCP	Received Signal Code Power

RSE	Radio System Entity

RSL	Radio Signalling Link

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RST	Reset

RSTD	Reference Signal Time Difference

RSVP 	Resource ReserVation Protocol

RSZI	Regional Subscription Zone Identity

RT	Real Time

RTE	Remote Terminal Emulator

RTP	Real Time Protocol

RU	Resource Unit

RWB	Resolution Bandwidth

RX	Receive

RXLEV	Received signal level

RXQUAL	Received Signal Quality

## S

S1AP	S1 Application Protocol

S1-MME	S1 for the control plane

S1-U	S1 for the user plane

S-Block	Supervisory Block

S-CCPCH	Secondary Common Control Physical Channel

S-CPICH	Secondary Common Pilot Channel

S-CSCF	Serving CSCF

S-GW	Serving Gateway

S-RNTI	SRNC Radio Network Temporary Identity

S-TMSI	SAE Temporary Mobile Station Identifier

SAAL	Signalling ATM Adaptation Layer

SABM	Set Asynchronous Balanced Mode

SACCH	Slow Associated Control Channel

SACCH/C4	Slow Associated Control CHannel/SDCCH/4

SACCH/C8	Slow Associated Control CHannel/SDCCH/8

SACCH/T	Slow Associated Control CHannel/Traffic channel

SACCH/TF	Slow Associated Control CHannel/Traffic channel Full rate

SACCH/TH	Slow Associated Control CHannel/Traffic channel Half rate

SAD	Source Address

SAE	System Architecture Evolution

SAP	Service Access Point

SAPI	Service Access Point Identifier

SAR	Segmentation and Reassembly

SAT	SIM Application Toolkit

SB	Synchronization  Burst

SBAS	Space Based Augmentation System

SBLP	Service Based Local Policy

SBSC	Serving Base Station Controller

SBSS	Serving Base Station Subsystem

SC	Service Centre (used for SMS)

Service Code

SC-FDMA	Single-Carrier Frequency Division Multiple Access

SCCH	Synchronisation Control Channel

SCCP	Signalling Connection Control Part

SCell	Secondary Cell

SCF	Service Control Function (IN context), Service Capability Feature (VHE/OSA context)

SCH	Synchronisation Channel

SCI	Subscriber Controlled Input

SCN	Sub-Channel Number

SCP	Service Control Point

SCTP	S Common Transport Protocol

SCUDIF	Service Change and UDI/RDI Fallback

SDCCH	Stand-Alone Dedicated Control Channel

SDH	Synchronous Digital Hierarchy

SDL	Specification Description Language

SDMA	Spatial Division Multiple Access

SDN	Service Dialling Number

SDP	Service Discovery Protocol (Bluetooth related)

Session Description Protocol

SDT	SDL Development Tool

SDU	Service Data Unit

SE	Security Environment

Sending Entity

Support Entity

SEF	Support Entity Function

SET	SUPL Enabled Terminal

SF	Spreading Factor

SFH	Slow Frequency Hopping

SFI	Short EF Identifier

SFN	System Frame Number

SGSN	Serving GPRS Support Node

SHCCH	Shared Channel Control Channel

SI	Screening Indicator

Service Interworking

Supplementary Information (SIA = Supplemenatary Information A)

System Information

SI-RNTI	System Information RNTI

SIB	System Information Block

SIC	Service Implementation Capabilities

SID	SIlence Descriptor

SIM	GSM Subscriber Identity Module

SIP	Session Initiated Protocol

SIPTO	Selected IP Traffic Offload

SIR	Signal-to-Interference Ratio

SLA	Service Level Agreement

SLP	SUPL Location Platform

SLPP	Subscriber LCS Privacy Profile

SLR	Send Loudness Rating

SLTM	Signalling Link Test Message

SM	Session Management

Short Message

SMDS	Switched Multimegabit Data Service

SME	Short Message Entity

SMG	Special Mobile Group

SMI	Structure of Management Information (RFC 1155)

SMLC	Serving Mobile Location Centre

SMS	Short Message Service

SMS-CB	SMS Cell Broadcast

SMS-PP	Short Message Service/Point-to-Point

SMS-SC	Short Message Service - Service Centre

Smt	Short message terminal

SN	Serial Number

Serving Network

Sequence Number

Subscriber Number

SNDCP	Sub-Network Dependent Convergence Protocol

SNMP	Simple Network Management Protocol

SNR	Serial NumbeR

Signal-to-Noise Ratio

SOA	Suppress Outgoing Access (CUG SS)

SoLSA	Support of Localised Service Area

SON	Self Organizing Networks

SoR	Steering of Roaming

SP	Switching Point

Service Provider

SPC	Signalling Point Code

Suppress Preferential CUG

SPCK	Service Provider Control Key

SPI	Security Parameters Indication

SQN	Sequence number

SR	Scheduling Request

SRB	Signalling Radio Bearer

SRES	Signed RESponse (authentication value returned by the SIM or by the USIM in 2G AKA)

SRNC	Serving Radio Network Controller

SRNS	Serving RNS

SRS	Sounding Reference Signal

SS	Supplementary Service

System Simulator

SS7	Signalling System No. 7

SSC	Secondary Synchronisation Code

Supplementary Service Control string

SSCOP	Service Specific Connection Oriented Protocol

SSCF	Service Specific Co-ordination Function

SSCF-NNI	Service Specific Coordination Function – Network Node Interface

SSCS	Service Specific Convergence Sublayer

SSDT	Site Selection Diversity Transmission

SSE	Service Specific Entities

SSF	Service Switching Function

SSN	Sub-System Number

SSSAR	Service Specific Segmentation and Re-assembly sublayer

STC	Signalling Transport Converter

STMR	SideTone Masking Rating

STP	Signalling Transfer Point

STTD	Space Time Transmit Diversity

SuM	Subscription Management

SUPL	Secure User Plane Location

SV	Space Vehicle

SVC	Switched virtual circuit

SVN	Software Version Number

SW	Status Word

Software

SW1/SW2	Status Word 1/Status Word 2

## T

T-SGW	Transport Signalling Gateway

T	Timer

Transparent

Type only

TA	Terminal Adaptation

Timing Advance

Tracking Area

TAC	Type Approval Code

TAF	Terminal Adaptation Function

TAR	Toolkit Application Reference

TB	Transport Block

TBD	To Be Defined

TBF	Temporary Block Flow

TBR	Technical Basis for Regulation

TC	Transaction Capabilities

TransCoder

Transmission Convergence

TCH	Traffic Channel

TCH/F	A full rate TCH

TCH/F2,4	A full rate data TCH (≤2,4kbit/s)

TCH/F4,8	A full rate date TCH (4,8kbit/s)

TCH/F9,6	A full rate data TCH (9,6kbit/s)

TCH/FS	A full rate Speech TCH

TCH/H	A half rate TCH

TCH/H2,4	A half rate data TCH (≤2,4kbit/s)

TCH/H4,8	A half rate data TCH (4,8kbit/s)

TCH/HS	A half rate Speech TCH

TC-TR	Technical Committee Technical Report

TCI	Transceiver Control Interface

TCP	Transmission Control Protocol

TD-CDMA	Time Division-Code Division Multiple Access

TDD	Time Division Duplex(ing)

TDMA	Time Division Multiple Access

TDoc	Temporary Document

TE	Terminal Equipment

TE9	Terminal Equipment 9 (ETSI sub-technical committee)

Tei	Terminal endpoint identifier

TEID	Tunnel End Point Identifier

TF	Transport Format

TFA	TransFer Allowed

TFC	Transport Format Combination

TFCI 	Transport Format Combination Indicator

TFCS	Transport Format Combination Set

TFI	Transport Format Indicator

Temporary Flow Identity

TFIN	Transport Format INdicator

TFP	TransFer Prohibited

TFS	Transport Format Set

TFT	Traffic Flow Template

TI	Transaction Identifier

TLLI	Temporary Logical Link Identity

TLM	TeLeMetry word

TLS	Transport Layer Security

TLV	Tag Length Value

TM	Telecom Management

Transparent Mode

TMA	Tower Mounted Amplifier

TMAAP	Tower Mounted Amplifier application part

TMF	Telecom Management Forum

TMN	Telecom Management Network

TMSI	Temporary Mobile Subscriber Identity

TN	Termination Node

Timeslot Number

TNL	Transport Network Layer

TO	Telecom Operations Map

TOA	Time of Arrival

TON	Type Of Number

TP	Third Party

TPC	Transmit Power Control

TPDU	Transfer Protocol Data Unit

TR	Technical Report

TRAU	Transcoder and Rate Adapter Unit

TrCH	Transport Channel

TRX	Transceiver

TS	Technical Specification

Teleservice

Time Slot

TSC	Training Sequence Code

TSDI	Transceiver Speech & Data Interface

TSG	Technical Specification Group

TSTD	Time Switched Transmit Diversity

TTCN	Tree and Tabular Combined Notation

TTI	Transmission Timing Interval

TUP	Telephone User Part (SS7)

TV	Type and Value

TX	Transmit

TXPWR	Transmit PoWeR; Tx power level in the MS_TXPWR_REQUEST and MS_TXPWR_CONF parameters

## U

U-plane	User plane

U-RNTI	UTRAN Radio Network Temporary Identity

UARFCN	UTRA Absolute Radio Frequency Channel Number

UARFN	UTRA Absolute Radio Frequency Number

UART	Universal Asynchronous Receiver and Transmitter

UCS2	Universal Character Set 2

UDD	Unconstrained Delay Data

UDI	Unrestricted Digital Information

UDP	User Datagram Protocol

UDUB	User Determined User Busy

UDCH	User-plane Dedicated CHannel

UE	User Equipment

UER	User Equipment with ODMA relay operation enabled

UEM	operating band Unwanted Emissions Mask

UI	User Interface

Unnumbered Information (Frame)

UIA	3G Integrity Algorithm

UIC	Union Internationale des Chemins de Fer

UL	Uplink (Reverse Link)

UL-SCH	Uplink Shared channel

UM	Unacknowledged Mode

UML	Unified Modelling Language

UMS	User Mobility Server

UMSC	UMTS Mobile Services Switching Centre

UMTS	Universal Mobile Telecommunications System

UNI	User-Network Interface

UP	User Plane

UPCMI	Uniform PCM Interface (13-bit)

UPE	User Plane Entity

UPD	Up to date

UpPTS	Uplink Pilot Timeslot

UPT	Universal Personal Telecommunication

URA	User Registration Area

UTRAN Registration Area

URAN	UMTS Radio Access Network

URB	User Radio Bearer

URI	Uniform Resource Identifier

URL	Uniform Resource Locator

USAT	USIM Application Toolkit

USB	Universal Serial Bus

USC	UE Service Capabilities

USCH	Uplink Shared Channel

USF	Uplink State Flag

USIM	Universal Subscriber Identity Module

USSD	Unstructured Supplementary Service Data

UT	Universal Time

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

UUI	User-to-User Information

UUS	Uu Stratum

User-to-User Signalling

## V

V	Value only

VA	Voice Activity factor

VAD	Voice Activity Detection

VAP	Videotex Access Point

VASP	Value Added Service Provider

VBR	Variable Bit Rate

VBS	Voice Broadcast Service

VC	Virtual Circuit

VGCS	Voice Group Call Service

VHE	Virtual Home Environment

VLR	Visitor Location Register

VMSC	Visited MSC

VoIP	Voice Over IP

VPLMN	Visited Public Land Mobile Network

VPN	Virtual Private Network

VRB	Virtual Resource Block

VSC	Videotex Service Centre

V(SD)	Send state variable

VTX host	The components dedicated to Videotex service

## W

WA	Wide Area

WAAS	Wide Area Augmentation System

WAE	Wireless Application Environment

WAP	Wireless Application Protocol

WBEM	Web Based Enterprise Management

WCDMA	Wideband Code Division Multiple Access

WDP	Wireless Datagram Protocol

WG	Working Group

WGS-84	World Geodetic System 1984

WIM	Wireless Identity Module

WIN	Wireless Intelligent Network

WLAN	Wireless Local Area Network

WLAN UE	WLAN User Equipment

WPA	Wrong Password Attempts (counter)

WS	Work Station

WSP	Wireless Session Protocol

WTA	Wireless Telephony Applications

WTAI	Wireless Telephony Applications Interface

WTDD	Wideband Time Division Duplexing

WTLS	Wireless Transport Layer Security

WTP	Wireless Transaction Protocol

WTX	Waiting Time eXtenstion

WWT	Work Waiting Time

WWW	World Wide Web

## X

X2-C	X2-Control plane

X2-U	X2-User plane

XID	eXchange IDentifier

XMAC	exXpected Message Authentication Code (calculated by the USIM application in 3G AKA)

XML	eXtensible Markup Language

XRES	EXpected user RESponse

## Y

<void>

## Z

ZC	Zone Code

# 5	Equations

| CPICHIor | The ratio of the received energy per PN chip of the CPICH to the total transmit power spectral density at the Node_B (SS) antenna connector. |
| --- | --- |
| DPCH | Average energy per PN chip for DPCH. |
| DPCHIor | The ratio of the transmit energy per PN chip of the DPCH to the total transmit power spectral density at the Node_B antenna connector. |
| DPCCHIor | The ratio of the transmit energy per PN chip of the DPCCH to the total transmit power spectral density at the Node B antenna connector. |
| DPDCHIor | The ratio of the transmit energy per PN chip of the DPDCH to the total transmit power spectral density at the Node B antenna connector. |
| Ec | Average energy per PN chip. |
| EcIor | The ratio of the average transmit energy per PN chip for different fields or physical channels to the total transmit power spectral density. |
| Fuw | Frequency of unwanted signal |
| Io | The total received power spectral density, including signal and interference, as measured at the UE antenna connector. |
| Ioac | The power spectral density of the adjacent frequency channel as measured at the UE antenna connector. |
| Ioc | The power spectral density of a band limited white noise source (simulating interference from cells, which are not defined in a test procedure) as measured at the UE antenna connector.The power spectral density of a band limited white noise source (simulating interference from other cells) as measured at the UE antenna connector. |
| Ior | The total transmit power spectral density of the Forward down link at the base stationNode_B antenna connector. |
| Ior | The received power spectral density of the down link as measured at the UE antenna connector. |
| Iouw | Unwanted signal power level. |
| OCNS | Average energy per PN chip for the OCNS. |
| OCNSIor | The ratio of the average transmit energy per PN chip for the OCNS to the total transmit power spectral density. |
| P−CCPCH | Average* energy per PN chip for P-CCPCH. |
| P−CCPCH EcIo | The ratio of the received P-CCPCH energy per chip to the total received power spectral density at the UE antenna connector. |
| P−CCPCHIor | The ratio of the average* transmit energy per PN chip for the P-CCPCH to the total transmit power spectral density. |
| P−CPICH | Average* energy per PN chip for P-CPICH. |
| PICH | Average* energy per PN chip for PICH. |
| PICHIor | The ratio of the received energy per PN chip of the PICH to the total transmit power spectral density at the Node B (SS) antenna connector. |
| PCCPCH EcIo | The ratio of the received PCCPCH energy per chip to the total received power spectral density at the UE antenna connector. |
| PCCPCHIor | The ratio of the average transmit energy per PN chip for the PCCPCH to the total transmit power spectral density. |
| DPCHIor | The ratio of the sum DPCH_Ex for one service in case of multicode to the total tramsmit power spectral density of the downlink at the BS antenna connector. |
| S−CCPCH | Average energy per PN chip for S-CCPCH. |
| S−CPICH | Average* energy per PN chip for S-CPICH. |
| SCH | Average* energy per PN chip for SCH. |
| SCCPCH | Average energy per PN chip for SCCPCH. |

*Note: Averaging period for energy/power of discontinuously transmitted channels should be defined.

Annex A:
Change history

| TSG SA# | SA Doc. | SA1 Doc | Spec | CR | Rev | Rel | Cat | Subject/Comment | Old | New | WI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SP-07 | - | - | 21.905 | - | - | - | - | Approved at SA#07 as version 3.0.0 |  | 3.0.0 |  |
| SP-08 | SP-000209 | S1-000369 | 21.905 | 0001 |  | R99 | B | New Abbreviations and Definitions for R99, language alignment and editorial changes | 3.0.0 | 3.1.0 |  |
| 08/2000 | - | - | 21.905 | - | - | - | - | MCC correction of CR001 implementation; editorial update. | 3.1.0 | 3.1.1 |  |
| SP-09 | SP-000380 | S1-000477 | 21.905 | 0002 |  | R99 | D | New Abbreviations and Definitions for R99 | 3.1.1 | 3.2.0 |  |
| SP-09 | SP-000381 | S1-000627 | 21.905 | 0003 |  | R4 | D | Change of Name of MExE | 3.1.1 | 4.0.0 |  |
| SP-10 | SP-000659 | S1-000731 | 21.905 | 0004 |  | Rel-4 | B | Introduces ASCI definition | 4.0.0 | 4.1.0 | ASCI |
| SP-10 | SP-000659 | S1-000736 | 21.905 | 0005 | 1 | Rel-4 | B | Inclusion of GSM 01.04 v 7.0.0 acronyms and abbreviations in the vocabulary | 4.0.0 | 4.1.0 | CORRECT |
| SP-11 | SP-010038 | S1-010233 | 21.905 | 0006 |  | Rel-4 | D | Editorial changes and new definitions | 4.1.0 | 4.2.0 | Vocab |
| SP-11 | SP-010038 | S1-010234 | 21.905 | 0007 |  | Rel-4 | B | Inclusion of commonly used definition contained in 23.122 | 4.1.0 | 4.2.0 | Vocab |
| SP-12 | SP-010256 | S1-010366 | 21.905 | 0008 |  | Rel-4 | F | Corrections to the vocabulary requested by RAN-4 | 4.2.0 | 4.3.0 | Vocab |
| SP-12 | SP-010256 | S1-010582 | 21.905 | 0009 |  | Rel-4 | F | CR to 21.905 on Definitions in 22.101 subscription and service provider | 4.2.0 | 4.3.0 | Vocab |
| SP-12 | SP-010258 | S1-010537 | 21.905 | 0010 |  | Rel-5 | D | Addition of definition of Service Provider and Subscription. Modification of definition of Subscriber | 4.3.0 | 5.0.0 | Vocab |
| SP-13 | SP-010430 | S1-010649 | 21.905 | 0013 |  | Rel-5 | B | CR to 21.905v5.0.0 (Rel-5) on Alignment of definitions requested by RAN 4 | 5.0.0 | 5.1.0 | Vocab |
| SP-13 | SP-010431 | S1-010838 | 21.905 | 0016 |  | Rel-5 | B | CR to 21.905 version 5.0.0 Nomenclature for GTT | 5.0.0 | 5.1.0 | GTT |
| SP-14 | SP-010671 | S1-011276 | 21.905 | 0021 | 1 | Rel-5 | F | Defintion of Local Services | 5.1.0 | 5.2.0 | IMS |
| SP-15 | SP-020046 | S1-020393 | 21.905 | 0030 |  | Rel-5 | B | CR to 21.905: new definition of the term 'service' | 5.2.0 | 5.3.0 | TEI |
| SP-15 | SP-020063 | S1-020431 | 21.905 | 0031 |  | Rel-5 | B | CR 21.905 Rel. 5 Introduction of new abbreviations derived of the approval of 3GPP TS 23.236 | 5.2.0 | 5.3.0 | PSS-E |
| SP-15 | SP-020046 | S1-020452 | 21.905 | 0032 |  | Rel-5 | B | CR 21.905 Rel.5 B Introduction of the definitions of “pre-pay” and “post-pay” billing | 5.2.0 | 5.3.0 | TEI |
| SP-15 | SP-020046 | S1-020526 | 21.905 | 0033 |  | Rel-5 | F | CR to 21.905: Replacement of the term UMTS with 3GPP system | 5.2.0 | 5.3.0 | TEI |
| SP-15 | SP-020046 | S1-020527 | 21.905 | 0034 |  | Rel-5 | B | CR to 21.905: missing abbreviations | 5.2.0 | 5.3.0 | TEI |
| SP-15 | SP-020046 | S1-020528 | 21.905 | 0035 |  | Rel-5 | B | CR to 21.905: new definition of the term 'application' | 5.2.0 | 5.3.0 | TEI |
| SP-15 | SP-020046 | S1-020617 | 21.905 | 0036 |  | Rel-5 | B | CR to 21905: definitions of online and offline charging | 5.2.0 | 5.3.0 | TEI |
| SP-15 | SP-020046 | S1-020620 | 21.905 | 0037 |  | Rel-5 | B | CR to 21.905: Improved definition of the term “application” | 5.2.0 | 5.3.0 | TEI |
| SP-16 | SP-020243 | S1-020973 | 21.905 | 0038 |  | Rel-5 | F | CR to 21.905 5.3.0 - removal of obsolete reference | 5.3.0 | 5.4.0 | Vocab |
| SP-17 | SP-020596 |  | 21.905 | 0039 | 1 | Rel-5 | F | Addition of GERAN definitions and abbreviations | 5.4.0 | 5.5.0 | TEI |
| SP-17 | SP-020596 |  | 21.905 | 0040 | 1 | Rel-5 | F | Addition of missing GSM/GPRS abbreviations | 5.4.0 | 5.5.0 | TEI |
| SP-17 | SP-020555 | S1-021762 | 21.905 | 0041 |  | Rel-6 | B | CR to 21.905 definitions from TR 22.951 | 5.4.0 | 6.0.0 | TEI |
| SP-17 | SP-020555 | S1-021715 | 21.905 | 0042 |  | Rel-6 | F | Enhancement of the definition of the 'Subscriber' | 5.4.0 | 6.0.0 | TEI |
| SP-18 | SP-020654 | S1-022223 | 21.905 | 0043 |  | Rel-6 | D | Update to 3GPP TR 21.905, Vocabulary for 3GPP Specifications | 6.0.0 | 6.1.0 | TEI6 |
| SP-18 | SP-020666 | S1-022264 | 21.905 | 0044 |  | Rel-6 | B | CR to 21.905 to introduce WLAN terminology | 6.0.0 | 6.1.0 | WLAN |
| SP-19 | SP-030012 | S1-030238 | 21.905 | 0046 | - | Rel-6 | A | CR on Entities of the mobile system | 6.1.0 | 6.2.0 | OAM-AR |
| SP-20 | SP-030247 | S1-030391 | 21.905 | 0047 | - | Rel-6 | B | Addition of the definition and acronym of 3GPP Generic User Profile | 6.2.0 | 6.3.0 | GUP |
| SP-20 | SP-030240 | S1-030576 | 21.905 | 0050 | - | Rel-6 | A | Correction of acronyms in TR21.905 | 6.2.0 | 6.3.0 | TEI4 |
| SP-21 | SP-030456 | S1-030971 | 21.905 | 0052 | - | Rel-6 | A | Correction of the Defintion of CDR | 6.3.0 | 6.4.0 | OAM-CH |
| SP-22 | SP-030694 | S1-031145 | 21.905 | 0053 | - | Rel-6 | F | Terminology addtions for IP-CAN and IP-CAN bearer | 6.4.0 | 6.5.0 | TEI6 |
| SP-22 | SP-030694 | S1-031311 | 21.905 | 0054 | - | Rel-6 | F | Modified base station definition | 6.4.0 | 6.5.0 | Vocab |
| SP-23 | SP-040087 | S1-040115 | 21.905 | 0055 | - | Rel-6 | B | Acronyms for the Flexible Layer One | 6.5.0 | 6.6.0 | FLOGER |
| SP-23 | SP-040107 | S5-042112 | 21.905 | 0056 | - | Rel-6 | F | Add Subscription Management (SuM) Definition and Abbreviation to SA1's 21.905 - Align with SA5's 32.140/1, 32.171/2/... & 3GPP Work Plan (WI Acronym) | 6.5.0 | 6.6.0 | SuM |
| SP-24 | SP-040286 | S1-040507 | 21.905 | 0057 | - | Rel-6 | F | Inclusion of ANP abbreviation as requested by SA3 | 6.6.0 | 6.7.0 | Vocab |
| SP-24 | SP-040476 | - | 21.905 | 0058 | 2 | Rel-6 | F | TR 21.905 Addition WLAN UE definition and classes of equipment and abbreviation | 6.6.0 | 6.7.0 | WLAN |
| SP-27 | SP-050055 | S1-050143 | 21.905 | 0061 | - | Rel-6 | A | Introduction of RAN Information Management | 6.7.0 | 6.8.0 | TEI5 |
| SP-28 | SP-050213 | S1-050487 | 21.905 | 0062 | - | Rel-6 | F | Correction of OSA acronym | 6.8.0 | 6.9.0 | Vocab |
| SP-29 | SP-050509 | S1-050780 | 21.905 | 0065 | - | Rel-6 | A | Abbreviation for SCUDIF | 6.9.0 | 6.10.0 | Vocab |
| SP-29 | SP-050515 | S1-050781 | 21.905 | 0066 | - | Rel-6 | F | Definition and abbreviation for DSAC | 6.9.0 | 6.10.0 | Vocab |
| SP-29 | SP-050524 | S1-050828 | 21.905 | 0067 | - | Rel-7 | B | Introduction of SBLP abbreviation | 6.9.0 | 7.0.0 | Vocab |
| SP-31 | SP-060033 | S1-060266 | 21.905 | 0068 | - | Rel-7 | F | Correction of terminology | 7.0.0 | 7.1.0 | NSP-CR |
| SP-32 | SP-060428 | - | 21.905 | 0069 | 1 | Rel-7 | F | TISPAN UE definition | 7.1.0 | 7.2.0 | FBI |
| SP-35 | SP-070231 | - | 21.905 | 0071 | 3 | Rel-7 | F | Terminology clarification for User Equipment and User Equipment components | 7.2.0 | 7.3.0 | Vocab |
| SP-35 | SP-070135 | S1-070248 | 21.905 | 0072 | - | Rel-8 | D | Adding FMC to terms and abbreviations | 7.3.0 | 8.0.0 | Vocab |
| SP-36 | SP-070475 | S1-070442 | 21.905 | 0074 | 1 | Rel-8 | A | Addition of "Steering of Roaming" to definitions and abbreviations | 8.0.0 | 8.1.0 | TEI |
| SP-37 | SP-070562 | S1-070949 | 21.905 | 76 |  | Rel-8 | B | To define 'Service Continuity' in the vocabulary | 8.1.0 | 8.2.0 | TEI8 |
| SP-37 | SP-070562 | S1-070986 | 21.905 | 77 |  | Rel-8 | B | Proposal to add E-UTRA and E-UTRAN | 8.1.0 | 8.2.0 | TEI8 |
| SP-37 | SP-070562 | S1-071102 | 21.905 | 75 | 1 | Rel-8 | B | Proposal to add Evolved Packet System Evolved Packet Core | 8.1.0 | 8.2.0 | TEI8 |
| SP-37 | SP-070562 | S1-071233 | 21.905 | 78 | 2 | Rel-8 | C | NP definition | 8.1.0 | 8.2.0 | TEI8 |
| SP-38 | SP-070848 | S1-071893 | 21.905 | 0079 | 1 | Rel-8 | B | Addition of definitions of an End-User and End-User Identity | 8.2.0 | 8.3.0 | EUI |
| SP-39 | SP-080045 | S1-080276 | 21.905 | 0080 | 2 | Rel-8 | F | Proposal to add abbreviation for Evolved Packet Core | 8.3.0 | 8.4.0 | TEI8 |
| SP-39 | SP-080045 | S1-080275 | 21.905 | 0081 | 2 | Rel-8 | F | Correction of UICC definition | 8.3.0 | 8.4.0 | TEI8 |
| SP-40 | SP-080298 | S1-080565 | 21.905 | 0082 | 1 | Rel-8 | B | Addition of definition of Pilot Identity | 8.4.0 | 8.5.0 | TEI8 |
| SP-41 | SP-080493 | S1-082395 | 21.905 | 0083 | 2 | Rel-8 | B | Add definitions and abbreviations related to Home NodeB and Home eNodeB | 8.5.0 | 8.6.0 | TEI8 |
| SP-42 | SP-080769 | S1-083441 | 21.905 | 0089 | 1 | Rel-9 | B | Addition of definition of IMS Credentials and IMC abbreviation | 8.6.0 | 9.0.0 | CIMS_3GPP2 |
| SP-43 | SP-090080 | S1-090167 | 21.905 | 0092 | 1 | Rel-9 | A | Introduce the definition of CSG manager (Mirror CR to rel-9) | 9.0.0 | 9.1.0 | HomeNB |
| SP-43 | SP-090081 | S1-090160 | 21.905 | 0094 | 2 | Rel-9 | A | Editorial changes in IMC definition | 9.0.0 | 9.1.0 | CIMS_3GPP2 |
| SP-44 | SP-090373 | S1-091277 | 21.905 | 0095 | 1 | Rel-9 | F | Align definition of Allowed CSG list | 9.1.0 | 9.2.0 | TEI-9 |
| SP-45 | SP-090484 | S1-093342 | 21.905 | 0098 | - | Rel-10 | B | Definition for Local IP Access and Selected IP Traffic Offload | 9.2.0 | 10.0.0 | LIPA_SIPTO |
| SP-46 | SP-090844 | S1-094274 | 21.905 | 0101 | 1 | Rel-10 | A | Clarify the term "Active Set" in 21.905 | 10.0.0 | 10.1.0 | TEI9 |
| SP-46 | SP-090848 | S1-094467 | 21.905 | 0102 | 3 | Rel-10 | B | Adding IFOM ( IP Flow Mobility) | 10.0.0 | 10.1.0 | IFOM |
| SP-47 | SP-100188 | S1-100438 | 21.905 | 0103 | 1 | Rel-10 | A | Adding definition of IMS Multimedia Telephony | 10.1.0 | 10.2.0 | TEI10 |
| SP-47 | SP-100189 | S1-100248 | 21.905 | 0104 | 4 | Rel-10 | B | Definition of Heterogeneous networks | 10.1.0 | 10.2.0 | TEI10 |
| SP-51 | SP-110091 |  | 21.905 | 105 | 4 | Rel-10 | F | Addition of new terms and abbreviations | 10.2.0 | 10.3.0 | TEI10 |